|  | SAFETY DATA SHEET |  |
| --- | --- | --- |
| Creation Date 13-Jan-2010 | Revision Date 11-Aug-2014 | Revision Number 1 |

# 1. Identification

| Product Name Adipic acid Cat No. : AC102810000; AC102810010; AC102810025; AC102810050; |  |
| --- | --- |
| AC102810250; AC102815000 Synonyms Hexanedioic acid Recommended Use Laboratory chemicals. Uses advised against No Information available Details of the supplier of the safety data sheet |  |
| Company Entity / Business Name Fisher Scientific Acros Organics One Reagent Lane One Reagent Lane Fair Lawn, NJ 07410 Fair Lawn, NJ 07410 Tel: (201) 796-7100 | Emergency Telephone Number For information US call: 001-800-ACROS-01 / Europe call: +32 14 57 52 11 Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99 CHEMTREC Tel. No.US:001-800-424-9300 / Europe:001-703-527-3887 |

# 2. Hazard(s) identification

## Classification

This chemical is considered hazardous by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200)

| Serious Eye Damage/Eye Irritation | Category 2 |
| --- | --- |
| Specific target organ toxicity (single exposure) Target Organs - Respiratory system. | Category 3 |

## Label Elements

Signal Word Warning

Hazard Statements

Causes serious eye irritation

May cause respiratory irritation

Precautionary Statements

______________________________________________________________________________________________

Page  1 / 7

Prevention

Wash face, hands and any exposed skin thoroughly after handling

Wear eye/face protection

Avoid breathing dust/fume/gas/mist/vapors/spray

Use only outdoors or in a well-ventilated area

Inhalation

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Call a POISON CENTER or doctor/physician if you feel unwell

Eyes

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing If eye irritation persists: Get medical advice/attention

Storage

Store in a well-ventilated place. Keep container tightly closed

Store locked up

Disposal

Dispose of contents/container to an approved waste disposal plant

Hazards not otherwise classified (HNOC)

None identified

# 3. Composition / information on ingredients

| Component |  |  | CAS-No | Weight % |  |
| --- | --- | --- | --- | --- | --- |
| Adipic acid |  |  | 124-04-9 | >95 |  |
|  |  | 4. First-aid measures |  |  |  |
| Eye Contact |  | Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Obtain medical attention. |  |  |  |
| Skin Contact |  | Wash off immediately with plenty of water for at least 15 minutes. Get medical attention if symptoms occur. |  |  |  |
| Inhalation |  | Move to fresh air. If breathing is difficult, give oxygen. Get medical attention if symptoms occur. |  |  |  |
| Ingestion |  | Do not induce vomiting. Obtain medical attention. |  |  |  |
| Most important symptoms/effects |  | No information available. |  |  |  |
| Notes to Physician |  | Treat symptomatically |  |  |  |
|  |  | 5. Fire-fighting measures |  |  |  |
| Suitable Extinguishing Media |  | Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. |  |  |  |
| Unsuitable Extinguishing Media |  | No information available |  |  |  |
| Flash Point |  | 196 °C / 384.8 °F |  |  |  |
| Method - |  | No information available |  |  |  |
| Autoignition Temperature Explosion Limits |  | 420 °C / 788 °F |  |  |  |
| Upper |  | No data available |  |  |  |
| Lower |  | No data available |  |  |  |

Sensitivity to Mechanical Impact No information available

Sensitivity to Static Discharge	No information available

Specific Hazards Arising from the Chemical

Thermal decomposition can lead to release of irritating gases and vapors. Keep product and empty container away from heat and sources of ignition.

Hazardous Combustion Products

Carbon monoxide (CO) Carbon dioxide (CO2)

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

## NFPA

Health Flammability Instability Physical hazards 2 1 0 N/A

|  |  | 6. Accidental release measures |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Personal Precautions |  | Use personal protective equipment. Ensure adequate ventilation. Avoid dust formation. Avoid contact with skin, eyes and clothing. |  |  |  |  |  |
| Environmental Precautions |  | Avoid release to the environment. See Section 12 for additional ecological information. |  |  |  |  |  |
| Methods for Containment an Up | d Clean Sweep up or vacuum up spillage and collect in suitable container for disposal. Avoid dust formation. |  |  |  |  |  |  |
|  | 7. Handling and storage |  |  |  |  |  |  |
| Handling | Wear personal protective equipment. Ensure adequate ventilation. Do not get in eyes, on skin, or on clothing. Avoid ingestion and inhalation. Avoid dust formation. |  |  |  |  |  |  |
| Storage | Keep containers tightly closed in a dry, cool and well-ventilated place. |  |  |  |  |  |  |
|  | 8. Exposure controls / personal protection |  |  |  |  |  |  |
| Exposure Guidelines |  |  |  |  |  |  |  |
| Component |  | ACGIH TLV |  |  | OSHA PEL | NIOSH IDLH |  |
| Adipic acid |  | TWA: 5 mg/m3 |  |  |  |  |  |
| Component |  | Quebec |  |  | Mexico OEL (TWA) | Ontario TWAEV |  |
| Adipic acid |  | TWA: 5 mg/m3 |  |  |  | TWA: 5 mg/m3 |  |
| Legend ACGIH - American Conference of Governm Engineering Measures Personal Protective Equipment Eye/face Protection Skin and body protection Respiratory Protection Hygiene Measures |  |  |  | ental Industrial Hygienists Ensure that eyewash stations and safety showers are close to the workstation location. Ensure adequate ventilation, especially in confined areas. Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. Wear appropriate protective gloves and clothing to prevent skin exposure. Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149. Use a NIOSH/MSHA or European Standard EN 149 approved respirator if exposure limits are exceeded or if irritation or other symptoms are experienced. Handle in accordance with good industrial hygiene and safety practice. |  |  |  |
|  |  | 9. Physical and chemical properties |  |  |  |  |  |
| Physical State |  |  |  | Solid |  |  |  |
| Appearance |  |  |  | White |  |  |  |
| Odor |  |  |  | Odorless |  |  |  |
| Odor Threshold |  |  |  | No information available |  |  |  |
| pH |  |  |  | 3.2 (0.1 % Solution) |  |  |  |
| Melting Point/Range |  |  |  | 151 - 153 °C / 303.8 - 307.4 °F |  |  |  |
| Boiling Point/Range |  |  |  | 265 °C / 509 °F @ 100 mmHg |  |  |  |
| Flash Point |  |  |  | 196 °C / 384.8 °F |  |  |  |
| Evaporation Rate |  |  |  | negligible |  |  |  |
| Flammability (solid,gas) Flammability or explosive limits |  |  |  | No information available |  |  |  |
| Upper |  |  |  | No data available |  |  |  |
| Lower |  |  |  | No data available |  |  |  |
| Vapor Pressure |  |  |  | <1 mbar @ 20 °C |  |  |  |

Vapor Density	5.04 (Air = 1.0)

Relative Density	No information available

Solubility	Insoluble in water

Partition coefficient; n-octanol/water	No data available

Autoignition Temperature	 420  °C  /  788  °F

Decomposition temperature	315 °C

Viscosity	No information available

Molecular Formula	C6 H10 O4 Molecular Weight	146.14

# 10. Stability and reactivity

Reactive Hazard	None known, based on information available

Stability	Stable under normal conditions.

Conditions to Avoid	Incompatible products. Excess heat. Avoid dust formation.

Incompatible Materials	Strong oxidizing agents

Hazardous Decomposition ProductsCarbon monoxide (CO), Carbon dioxide (CO2)

| Hazardous Polymerization | Hazardous polymerization does not occur. |
| --- | --- |
| Hazardous Reactions | None under normal processing. |

# 11. Toxicological information

| Acute Toxicity |  |
| --- | --- |
| Product Information Component Information | No acute toxicity information is available for this product |
| Toxicologically Synergistic | No information available |

Products

## Delayed and immediate effects as well as chronic effects from short and long-term exposure

| Irritation |  | Irritating to eyes and respiratory system |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sensitization |  | No information available |  |  |  |  |
| Carcinogenicity |  | The table below indicates whether each agency has listed any ingredient as a carcinogen. |  |  |  |  |
| Component | CAS-No | IARC | NTP | ACGIH | OSHA | Mexico |
| Adipic acid | 124-04-9 | Not listed | Not listed | Not listed | Not listed | Not listed |

Mutagenic Effects	No information available

Reproductive Effects	No information available.

Developmental Effects	No information available. Teratogenicity	No information available.

STOT - single exposure	Respiratory system STOT - repeated exposure	None known

Aspiration hazard	No information available

Symptoms  / effects,both acute and No information available delayed

Endocrine Disruptor Information	No information available

Other Adverse Effects	See actual entry in RTECS for complete information.

| 12. Ecological information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ecotoxicity . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component | Freshwater Algae |  |  |  |  | Freshwater Fish |  |  |  |  | Microtox |  |  |  | Water Flea |  |  |
| Adipic acid | EC50= 31.3 mg/L 72h EC50 = 26.6 mg/L 96h |  |  |  |  | Leuciscus idus: LC50 = 230 mg/L 96h |  |  |  |  | EC50 = 91.9 mg/L 17 h |  |  |  | EC50 = 85.7 mg/L 48h |  |  |
| Persistence and Degradability |  |  |  | No information available |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bioaccumulation/ Accumulation |  |  |  | No information available. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mobility |  |  |  | . |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Component |  |  |  |  |  | log Pow |  |  |  |  |  |  |  |  |  |
|  |  | Adipic acid |  |  |  |  |  | 0.081 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 13. Disposal considerations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Waste Disposal Methods |  |  |  | Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. Chemical waste generators must also consult local, regional, and national hazardous waste regulations to ensure complete and accurate classification. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14. Transport information |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DOT |  |  |  | Not regulated |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDG |  |  |  | Not regulated |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IATA |  |  |  | Not regulated |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IMDG/IMO |  |  |  | Not regulated |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 15. Regulatory information |  |  |  |  |  |  |  |  |  |  |  |  |  |
| International Inventories |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component |  |  | TSCA | DSL | NDSL |  | EINECS |  | ELINCS | NLP |  | PICCS | ENCS | AICS |  | IECSC | KECL |
| Adipic acid |  |  | X | X | - |  | 204-673-3 |  | - |  |  | X | X | X |  | X | X |

Legend:

X - Listed

- - Indicates a substance that is the subject of a Section 5(e) Consent order under TSCA.

- - Indicates a substance that is the subject of a Section 5(f) Rule under TSCA.

N - Indicates a polymeric substance containing no free-radical initiator in its inventory name but is considered to cover the designated polymer made with any free-radical initiator regardless of the amount used.

P - Indicates a commenced PMN substance

R - Indicates a substance that is the subject of a Section 6 risk management rule under TSCA. S - Indicates a substance that is identified in a proposed or final Significant New Use Rule T - Indicates a substance that is the subject of a Section 4 test rule under TSCA.

XU - Indicates a substance exempt from reporting under the Inventory Update Rule, i.e. Partial Updating of the TSCA Inventory Data Base Production and Site Reports (40 CFR 710(B).

Y1 - Indicates an exempt polymer that has a number-average molecular weight of 1,000 or greater.

Y2 - Indicates an exempt polymer that is a polyester and is made only from reactants included in a specified list of low concern reactants that comprises one of the eligibility criteria for the exemption rule.

## U.S. Federal Regulations

| TSCA 12(b) Not applicable SARA 313 Not applicable SARA 311/312 Hazardous Categorization |  |
| --- | --- |
| Acute Health Hazard | Yes |
| Chronic Health Hazard | No |
| Fire Hazard | No |
| Sudden Release of Pressure Hazard | No |
| Reactive Hazard | No |

Clean Water Act

| Component | CWA - Hazardous Substances | CWA - Reportable Quantities | CWA - Toxic Pollutants | CWA - Priority Pollutants |
| --- | --- | --- | --- | --- |
| Adipic acid | X | 5000 lb | - | - |

Clean Air Act	Not applicable

OSHA Occupational Safety and Health Administration Not applicable

CERCLA

This material, as supplied, contains one or more substances regulated as a hazardous substance under the Comprehensive

Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302)

| Component |  |  | Hazardous Substances RQs |  |  | CERCLA EHS RQs |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Adipic acid |  |  | 5000 lb |  |  | - |  |
| California Proposition 65 This product does not contain any Proposition 65 ch State Right-to-Know |  |  |  |  |  | emicals |  |
| Component | Massachusetts | New Jersey |  | Pennsylvania |  | Illinois | Rhode Island |
| Adipic acid | X | X |  | X |  | - | X |

U.S. Department of Transportation

Reportable Quantity (RQ):	Y

DOT Marine Pollutant	N DOT Severe Marine Pollutant	N

U.S. Department of Homeland Security

This product does not contain any DHS chemicals.

## Other International Regulations

Mexico - Grade	Slight risk, Grade 1

Canada

This product has been classified in accordance with the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all the information required by the CPR

WHMIS Hazard Class	D2B  Toxic materials

# 16. Other information

| Prepared By | Regulatory Affairs Thermo Fisher Scientific Email: EMSDS.RA@thermofisher.com |
| --- | --- |
| Creation Date | 13-Jan-2010 |
| Revision Date | 11-Aug-2014 |
| Print Date | 11-Aug-2014 |
| Revision Summary | This document has been updated to comply with the US OSHA HazCom 2012 Standard replacing the current legislation under 29 CFR 1910.1200 to align with the Globally Harmonized System of Classification and Labeling of Chemicals (GHS) |

Disclaimer

The information provided on this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication.  The information given is designed only as a guide for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered as a warranty or quality specification.  The information relates only to the specific material designated and may not be valid for such material used in combination with any other material or in any process, unless specified in the text.

Adipic acid	Revision Date  11-Aug-2014

______________________________________________________________________________________________

# End of SDS

ADIPIC ACID

|  | Warning. |
| --- | --- |

Causes serious eye irritation

May cause respiratory irritation

Avoid breathing dust/fume/gas/mist/vapours/spray

Wash ... thoroughly after handling

Use only outdoors or with adequate ventilation

Wear protective gloves/protective clothing/eye protection/face protection

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing IF IN EYES: Rinse cautiously with water for several minutes,  Remove contact lenses,  if present and easy to do. Continue rinsing

Call a POISON CENTER or doctor/physician

If eye irritation persists: Get medical advice/attention

Store in a well-ventilated place,  Keep container tightly closed

Store locked up

Dispose of contents/container to ...

Acros Organics

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.

|  | SAFETY DATA SHEET |  |
| --- | --- | --- |
| Creation Date 30-Jun-2009 | Revision Date 17-Nov-2014 | Revision Number 1 |

- Identification

| Product Name Nitric acid, 20-64% Cat No. : AC133620000; AC133620010; AC133620011; AC133620025; |  |
| --- | --- |
| AC133620026 Synonyms No information available Recommended Use Laboratory chemicals. Uses advised against No Information available Details of the supplier of the safety data sheet |  |
| Company Entity / Business Name Fisher Scientific Acros Organics One Reagent Lane One Reagent Lane Fair Lawn, NJ 07410 Fair Lawn, NJ 07410 Tel: (201) 796-7100 | Emergency Telephone Number For information US call: 001-800-ACROS-01 / Europe call: +32 14 57 52 11 Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99 CHEMTREC Tel. No.US:001-800-424-9300 / Europe:001-703-527-3887 |

- Hazard(s) identification

Classification

This chemical is considered hazardous by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200)

| Corrosive to metals | Category 1 |
| --- | --- |
| Acute Inhalation Toxicity - Vapors | Category 4 |
| Skin Corrosion/irritation | Category 1 A |
| Serious Eye Damage/Eye Irritation | Category 1 |
| Specific target organ toxicity (single exposure) Target Organs - Respiratory system. | Category 3 |

Label Elements

Signal Word Danger

# Hazard Statements

May be corrosive to metals

Harmful if inhaled

Causes severe skin burns and eye damage

Causes serious eye damage

May cause respiratory irritation

# Precautionary Statements Prevention

Keep only in original container

Avoid breathing dust/fume/gas/mist/vapors/spray

Use only outdoors or in a well-ventilated area

Wash face, hands and any exposed skin thoroughly after handling

Wear protective gloves/protective clothing/eye protection/face protection

Response

Call a POISON CENTER or doctor/physician if you feel unwell

# Inhalation

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Call a POISON CENTER or doctor/physician if you feel unwell

# Skin

IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower

Immediately call a POISON CENTER or doctor/physician

Wash contaminated clothing before reuse Eyes

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing Immediately call a POISON CENTER or doctor/physician

Ingestion

IF SWALLOWED: Rinse mouth.  DO NOT induce vomiting

# Storage

Store locked up

Store in a well-ventilated place. Keep container tightly closed

Store in corrosive resistant polypropylene container with a resistant inliner

# Disposal

Dispose of contents/container to an approved waste disposal plant

Hazards not otherwise classified (HNOC)

None identified

- Composition / information on ingredients

| Component | CAS-No | Weight % |
| --- | --- | --- |
| Water | 7732-18-5 | 36-80 |
| Nitric acid | 7697-37-2 | 20-64 |

- First-aid measures

| Eye Contact | Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required. |
| --- | --- |
| Skin Contact | Wash off immediately with plenty of water for at least 15 minutes. Immediate medical attention is required. |
| Inhalation | Move to fresh air. If breathing is difficult, give oxygen. Do not use mouth-to-mouth resuscitation if victim ingested or inhaled the substance; induce artificial respiration with a respiratory medical device. Immediate medical attention is required. |
| Ingestion | Do not induce vomiting. Call a physician or Poison Control Center immediately. |
| Most important symptoms/effects | Causes burns by all exposure routes. Product is a corrosive material. Use of gastric lavage or emesis is contraindicated. Possible perforation of stomach or esophagus should be investigated: Ingestion causes severe swelling, severe damage to the delicate tissue and danger of perforation |  |
| Notes to Physician | Treat symptomatically |  |

- Fire-fighting measures

| Suitable Extinguishing Media | Substance is nonflammable; use agent most appropriate to extinguish surrounding fire. |
| --- | --- |
| Unsuitable Extinguishing Media | No information available |
| Flash Point | Not applicable |
| Method - | No information available |
| Autoignition Temperature Explosion Limits | No information available |
| Upper | No data available |
| Lower | No data available |

Sensitivity to Mechanical Impact No information available Sensitivity to Static Discharge	No information available

# Specific Hazards Arising from the Chemical

Corrosive Material. Causes severe burns by all exposure routes. Thermal decomposition can lead to release of irritating gases and vapors. Keep product and empty container away from heat and sources of ignition.

Hazardous Combustion Products

Nitrogen oxides (NOx)

# Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

NFPA

# Health	Flammability	Instability	Physical hazards

3	0	0	-

|  |  | 6. Accidental release measures |
| --- | --- | --- |
| Personal Precautions |  | Wear self-contained breathing apparatus and protective suit. Evacuate personnel to safe areas. Ensure adequate ventilation. Do not get in eyes, on skin, or on clothing. Use personal protective equipment. |
| Environmental Precautions |  | Avoid release to the environment. See Section 12 for additional ecological information. |
| Methods for Containment an Up | d Clean Wear self-contained breathing apparatus and protective suit. Soak up with inert absorbent material. Keep in suitable, closed containers for disposal. |  |
|  | 7. Handling and storage |  |
| Handling | Use only under a chemical fume hood. Wear personal protective equipment. Do not get in eyes, on skin, or on clothing. Do not breathe vapors or spray mist. Do not ingest. Contents under pressure. |  |
| Storage | Keep containers tightly closed in a dry, cool and well-ventilated place. Corrosives area. |  |
|  | 8. Exposure controls / personal protection |  |

Exposure Guidelines

| Component | ACGIH TLV | OSHA PEL | NIOSH IDLH |
| --- | --- | --- | --- |
| Nitric acid | TWA: 2 ppm STEL: 4 ppm | (Vacated) TWA: 2 ppm (Vacated) TWA: 5 mg/m3 (Vacated) STEL: 4 ppm (Vacated) STEL: 10 mg/m3 TWA: 2 ppm TWA: 5 mg/m3 | IDLH: 25 ppm TWA: 2 ppm TWA: 5 mg/m3 STEL: 4 ppm STEL: 10 mg/m3 |
| Component | Quebec | Mexico OEL (TWA) | Ontario TWAEV |
| Nitric acid | TWA: 2 ppm TWA: 5.2 mg/m3 STEL: 4 ppm STEL: 10 mg/m3 | TWA: 2 ppm TWA: 5 mg/m3 STEL: 4 ppm STEL: 10 mg/m3 | TWA: 2 ppm STEL: 4 ppm |

Legend

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

NIOSH IDLH:  The National Institute for Occupational Safety and Health Immediately Dangerous to Life or Health

| Engineering Measures Personal Protective Equipment | Use only under a chemical fume hood. Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations and safety showers are close to the workstation location. |
| --- | --- |
| Eye/face Protection | Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. |
| Skin and body protection | Wear appropriate protective gloves and clothing to prevent skin exposure. |
| Respiratory Protection | Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149. Use a NIOSH/MSHA or European Standard EN 149 approved respirator if exposure limits are exceeded or if irritation or other symptoms are experienced. |
| Hygiene Measures | When using, do not eat, drink or smoke. Remove and wash contaminated clothing before re-use. Provide regular cleaning of equipment, work area and clothing. |

- Physical and chemical properties

| Physical State | Liquid |
| --- | --- |
| Appearance | No information available |
| Odor | No information available |
| Odor Threshold | No information available |
| pH | No information available |
| Melting Point/Range | -42 °C / -43.6 °F |
| Boiling Point/Range | 122 °C / 251.6 °F |
| Flash Point | Not applicable |
| Evaporation Rate | No information available |
| Flammability (solid,gas) Flammability or explosive limits | Not applicable |
| Upper | No data available |
| Lower | No data available |
| Vapor Pressure | 0.94 kPa @ 20 °C |
| Vapor Density | No information available |
| Relative Density | 1.33 |
| Solubility | No information available |
| Partition coefficient; n-octanol/water | No data available |
| Autoignition Temperature | No information available |
| Decomposition Temperature | No information available |
| Viscosity | No information available |

- Stability and reactivity

| Reactive Hazard | None known, based on information available |
| --- | --- |
| Stability | Stable under normal conditions. |
| Conditions to Avoid | Incompatible products. Combustible material. Excess heat. |
| Incompatible Materials | Strong bases, Reducing agents, Organic materials, Aldehydes, Alcohols, Cyanides, Metals, Powdered metals, Ammonia |

# Hazardous Decomposition ProductsNitrogen oxides (NOx)

Hazardous Polymerization	Hazardous polymerization does not occur.

Hazardous Reactions	None under normal processing.

|  | 11. Toxicological information |  |  |
| --- | --- | --- | --- |
| Acute Toxicity Product Information Oral LD50 | Based on ATE data, the classification criteria are not met. ATE > 2000 mg/kg. |  |  |
| Dermal LD50 | Based on ATE data, the classification criteria are not met. ATE > 2000 mg/kg. |  |  |
| Vapor LC50 Component Information | Category 4. |  |  |
| Component | LD50 Oral | LD50 Dermal | LC50 Inhalation |
| Nitric acid | Not listed | Not listed | 130 mg/m3 ( Rat ) 4 h 67 ppm ( Rat ) 4 h |

# Toxicologically Synergistic	No information available Products

Delayed and immediate effects as well as chronic effects from short and long-term exposure

| Irritation |  |  | Causes burns by all exposure routes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sensitization |  |  | No information available |  |  |  |  |  |
| Carcinogenicity |  |  | The table below indicates whether each agency has listed any ingredient as a carcinogen. |  |  |  |  |  |
| Component | CAS-No |  | IARC | NTP | ACGIH | OSHA | Mexico |  |
| Water | 7732-18-5 |  | Not listed | Not listed | Not listed | Not listed | Not listed |  |
| Nitric acid | 7697-37-2 |  | Not listed | Not listed | Not listed | Not listed | Not listed |  |

Mutagenic Effects	No information available

Reproductive Effects	No information available.

Developmental Effects	No information available. Teratogenicity	No information available.

# STOT - single exposure	Respiratory system STOT - repeated exposure	None known

Aspiration hazard	No information available

Symptoms  / effects,both acute and Product is a corrosive material.  Use of gastric lavage or emesis is contraindicated.

delayed	Possible perforation of stomach or esophagus should be investigated: Ingestion causes

severe swelling, severe damage to the delicate tissue and danger of perforation Endocrine Disruptor Information	No information available

Other Adverse Effects	See actual entry in RTECS for complete information.

12. Ecological information

Ecotoxicity

The product contains following substances which are hazardous for the environment.

| Component | Freshwater Algae |  | Freshwater Fish |  | Microtox | Water Flea |
| --- | --- | --- | --- | --- | --- | --- |
| Nitric acid | Not listed |  | 72 mg/L LC50 96 h |  | Not listed | Not listed |
| Persistence and Degradability |  | Soluble in water Persistence is unlikely based on information available. |  |  |  |  |
| Bioaccumulation/ Accumulation |  | No information available. |  |  |  |  |
| Mobility |  | . Will likely be mobile in the environment due to its water solubility. |  |  |  |  |
| Component |  |  |  | log Pow |  |  |
| Nitric acid |  |  |  | -2.3 |  |  |
|  |  | 13. Disposal considerations |  |  |  |  |
| Waste Disposal Methods |  | Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. Chemical waste generators must also consult local, regional, and national hazardous waste regulations to ensure complete and accurate classification. |  |  |  |  |
|  |  | 14. Transport information |  |  |  |  |
| DOT UN-No |  | UN2031 |  |  |  |  |
| Proper Shipping Name |  | NITRIC ACID |  |  |  |  |
| Hazard Class |  | 8 |  |  |  |  |
| Packing Group |  | II |  |  |  |  |
| TDG UN-No |  | UN2031 |  |  |  |  |
| Proper Shipping Name |  | NITRIC ACID |  |  |  |  |
| Hazard Class |  | 8 |  |  |  |  |
| Packing Group |  | II |  |  |  |  |
| IATA UN-No |  | UN2031 |  |  |  |  |
| Proper Shipping Name |  | NITRIC ACID |  |  |  |  |
| Hazard Class |  | 8 |  |  |  |  |
| Packing Group |  | II |  |  |  |  |
| IMDG/IMO UN-No |  | UN2031 |  |  |  |  |
| Proper Shipping Name |  | NITRIC ACID |  |  |  |  |
| Hazard Class |  | 8 |  |  |  |  |
| Packing Group |  | II |  |  |  |  |
|  |  | 15. Regulatory information |  |  |  |  |

# International Inventories

| Component | TSCA | DSL | NDSL | EINECS | ELINCS | NLP | PICCS | ENCS | AICS | IECSC | KECL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water | X | X | - | 231-791-2 | - |  | X | - | X | X | X |
| Nitric acid | X | X | - | 231-714-2 | - |  | X | X | X | X | X |

Legend:

X - Listed

- - Indicates a substance that is the subject of a Section 5(e) Consent order under TSCA.

- - Indicates a substance that is the subject of a Section 5(f) Rule under TSCA.

N - Indicates a polymeric substance containing no free-radical initiator in its inventory name but is considered to cover the designated polymer made with any free-radical initiator regardless of the amount used.

P - Indicates a commenced PMN substance

R - Indicates a substance that is the subject of a Section 6 risk management rule under TSCA. S - Indicates a substance that is identified in a proposed or final Significant New Use Rule T - Indicates a substance that is the subject of a Section 4 test rule under TSCA.

XU - Indicates a substance exempt from reporting under the Inventory Update Rule, i.e. Partial Updating of the TSCA Inventory Data Base Production and Site Reports (40 CFR 710(B).

Y1 - Indicates an exempt polymer that has a number-average molecular weight of 1,000 or greater.

Y2 - Indicates an exempt polymer that is a polyester and is made only from reactants included in a specified list of low concern reactants that comprises one of the eligibility criteria for the exemption rule.

U.S. Federal Regulations

TSCA 12(b)	Not applicable

# SARA 313

| Component |  |  |  |  | CAS-No |  | Weight % |  |  | SARA 313 - Threshold Values % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nitric acid |  |  |  |  | 7697-37-2 |  | 20-64 |  |  | 1.0 |
| SARA 311/312 Hazardous Categorization Acute Health Hazard |  |  |  |  | Yes |  |  |  |  |  |
| Chronic Health Hazard |  |  |  |  | No |  |  |  |  |  |
| Fire Hazard |  |  |  |  | No |  |  |  |  |  |
| Sudden Release of Pressure Hazard |  |  |  |  | No |  |  |  |  |  |
| Reactive Hazard Clean Water Act |  |  |  |  | No |  |  |  |  |  |
| Component | CWA - Hazardous Substances |  |  |  | CWA - Reportable Quantities |  | CWA - Toxic Pollutants |  |  | CWA - Priority Pollutants |
| Nitric acid | X |  |  |  | 1000 lb |  | - |  |  | - |
| Clean Air Act OSHA Occupational Safety and Health Administr Not applicable | Not applicable ation |  |  |  |  |  |  |  |  |  |
| Component |  |  |  | Specifically Regulated Chemicals |  |  |  | Highly Hazardous Chemicals |  |  |
| Nitric acid |  |  |  | - |  |  |  | TQ: 500 lb |  |  |

# CERCLA

This material, as supplied, contains one or more substances regulated as a hazardous substance under the Comprehensive

Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302)

| Component | Hazardous Substances RQs | CERCLA EHS RQs |
| --- | --- | --- |
| Nitric acid | 1000 lb | 1000 lb |

California Proposition 65	This product does not contain any Proposition 65 chemicals State Right-to-Know

| Component | Massachusetts | New Jersey | Pennsylvania | Illinois | Rhode Island |
| --- | --- | --- | --- | --- | --- |
| Water | - | - | X | - | - |
| Nitric acid | X | X | X | X | X |

# U.S. Department of Transportation

Reportable Quantity (RQ):	Y

DOT Marine Pollutant	N DOT Severe Marine Pollutant	N

# U.S. Department of Homeland Security

This product contains the following DHS chemicals:

| Component | DHS Chemical Facility Anti-Terrorism Standard |
| --- | --- |
| Nitric acid | 2000 lb STQ |

Other International Regulations

Mexico - Grade	No information available

# Canada This product has been classified in accordance with the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all the information required by the CPR

WHMIS Hazard Class	E    Corrosive material

16. Other information

| Prepared By | Regulatory Affairs Thermo Fisher Scientific Email: EMSDS.RA@thermofisher.com |
| --- | --- |
| Creation Date | 30-Jun-2009 |
| Revision Date | 17-Nov-2014 |
| Print Date | 17-Nov-2014 |
| Revision Summary | This document has been updated to comply with the US OSHA HazCom 2012 Standard replacing the current legislation under 29 CFR 1910.1200 to align with the Globally Harmonized System of Classification and Labeling of Chemicals (GHS) |

# Disclaimer

The information provided on this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication.  The information given is designed only as a guide for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered as a warranty or quality specification.  The information relates only to the specific material designated and may not be valid for such material used in combination with any other material or in any process, unless specified in the text.

# End of SDS

NITRIC ACID, 20-64%

|  | Danger. |
| --- | --- |

Causes serious eye damage

Causes severe skin burns and eye damage

Harmful if inhaled

May be corrosive to metals

May cause respiratory irritation

Avoid breathing dust/fume/gas/mist/vapours/spray

Wash ... thoroughly after handling

Use only outdoors or with adequate ventilation

Wear protective gloves/protective clothing/eye protection/face protection

IF SWALLOWED: Rinse mouth,  Do NOT induce vomiting

IF ON SKIN (or hair): Take off immediately all contaminated clothing,  Rinse skin with water/shower

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing IF IN EYES: Rinse cautiously with water for several minutes,  Remove contact lenses,  if present and easy to do. Continue rinsing

Immediately call a POISON CENTER/doctor/…

Call a POISON CENTER or doctor/physician

Wash contaminated clothing before reuse

Store in accordance with…

Store in a well-ventilated place,  Keep container tightly closed

Store locked up

Dispose of contents/container to ...

Acros Organics

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.

|  | SAFETY DATA SHEET |  |
| --- | --- | --- |
| Creation Date 18-Jul-2014 | Revision Date 18-Jul-2014 | Revision Number 1 |

# 1. Identification

| Product Name Cyclohexanol Cat No. : AC147680000; AC147680010; AC147680025; AC147680050; |  |
| --- | --- |
| AC147680100 Synonyms Hexalin; Adronal; Cyclohexyl alcohol Recommended Use Laboratory chemicals. Uses advised against No Information available Details of the supplier of the safety data sheet |  |
| Company Entity / Business Name Fisher Scientific Acros Organics One Reagent Lane One Reagent Lane Fair Lawn, NJ 07410 Fair Lawn, NJ 07410 Tel: (201) 796-7100 | Emergency Telephone Number For information US call: 001-800-ACROS-01 / Europe call: +32 14 57 52 11 Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99 CHEMTREC Tel. No.US:001-800-424-9300 / Europe:001-703-527-3887 |

# 2. Hazard(s) identification

## Classification

This chemical is considered hazardous by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200)

| Flammable liquids | Category 4 |
| --- | --- |
| Acute oral toxicity | Category 4 |
| Acute Inhalation Toxicity - Vapors | Category 4 |
| Skin Corrosion/irritation | Category 2 |
| Serious Eye Damage/Eye Irritation | Category 2 |
| Specific target organ toxicity (single exposure) Target Organs - Respiratory system. | Category 3 |

## Label Elements

Signal Word Warning

### Hazard Statements

Combustible liquid

Harmful if swallowed

Harmful if inhaled

Causes skin irritation

Causes serious eye irritation

May cause respiratory irritation

______________________________________________________________________________________________

Page  1 / 7

### Precautionary Statements Prevention

Wash face, hands and any exposed skin thoroughly after handling

Do not eat, drink or smoke when using this product

Wear protective gloves/protective clothing/eye protection/face protection

Avoid breathing dust/fume/gas/mist/vapors/spray

Use only outdoors or in a well-ventilated area

Keep away from heat/sparks/open flames/hot surfaces. - No smoking

Keep cool

### Inhalation

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Call a POISON CENTER or doctor/physician if you feel unwell

### Skin

IF ON SKIN: Wash with plenty of soap and water

If skin irritation occurs: Get medical advice/attention

Take off contaminated clothing and wash before reuse

### Eyes

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing If eye irritation persists: Get medical advice/attention

### Ingestion

IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell Rinse mouth

Fire

In case of fire: Use CO2, dry chemical, or foam for extinction

### Storage

Store in a well-ventilated place. Keep container tightly closed

Store locked up

Disposal

Dispose of contents/container to an approved waste disposal plant

## Hazards not otherwise classified (HNOC)

None identified

| 3. Composition / information on ingredients |  |  |  |
| --- | --- | --- | --- |
| Component | CAS-No |  | Weight % |
| Cyclohexanol | 108-93-0 |  | 98 |
| 4. First-aid measures |  |  |  |

Eye Contact	Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes.

Obtain medical attention.

Skin Contact	Wash off immediately with plenty of water for at least 15 minutes. Get medical attention if symptoms occur.

Inhalation	Move to fresh air. If breathing is difficult, give oxygen. Do not use mouth-to-mouth

resuscitation if victim ingested or inhaled the substance; induce artificial respiration with a respiratory medical device. Obtain medical attention.

Ingestion	Do not induce vomiting. Obtain medical attention.

Most important symptoms/effects	Breathing difficulties.  Symptoms of overexposure may be headache, dizziness, tiredness, nausea and vomiting Notes to Physician	Treat symptomatically

# 5. Fire-fighting measures

| Suitable Extinguishing Media | Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. |
| --- | --- |
| Unsuitable Extinguishing Media | No information available |
| Flash Point | 67 °C / 152.6 °F |
| Method - | No information available |
| Autoignition Temperature Explosion Limits | 300 °C / 572 °F |
| Upper | No data available |
| Lower | 1.2 vol % |

Sensitivity to Mechanical Impact No information available Sensitivity to Static Discharge	No information available

## Specific Hazards Arising from the Chemical

Combustible material. Containers may explode when heated. Thermal decomposition can lead to release of irritating gases and vapors. Keep product and empty container away from heat and sources of ignition.

Hazardous Combustion Products

Carbon monoxide (CO) Carbon dioxide (CO2)

## Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

## NFPA

| Health 1 | Flammability Instability 2 0 | Physical hazards N/A |
| --- | --- | --- |
|  | 6. Accidental release measures |  |

Personal Precautions	Use personal protective equipment. Remove all sources of ignition. Avoid contact with skin, eyes and clothing.

Environmental Precautions	Avoid release to the environment. See Section 12 for additional ecological information.

Methods for Containment and Clean Remove all sources of ignition. Soak up with inert absorbent material. Keep in suitable,

Up	closed containers for disposal.

|  | 7. Handling and storage |
| --- | --- |
| Handling | Use only under a chemical fume hood. Use explosion-proof equipment. Avoid contact with skin, eyes and clothing. Avoid ingestion and inhalation. Keep away from open flames, hot surfaces and sources of ignition. |
| Storage | Keep in a dry, cool and well-ventilated place. Keep container tightly closed. Keep away from heat and sources of ignition. |
|  | 8. Exposure controls / personal protection |

## Exposure Guidelines

| Component | ACGIH TLV | OSHA PEL | NIOSH IDLH |
| --- | --- | --- | --- |
| Cyclohexanol | TWA: 50 ppm Skin | (Vacated) TWA: 50 ppm (Vacated) TWA: 200 mg/m3 Skin TWA: 50 ppm TWA: 200 mg/m3 | IDLH: 400 ppm TWA: 50 ppm TWA: 200 mg/m3 |
| Component | Quebec | Mexico OEL (TWA) | Ontario TWAEV |
| Cyclohexanol | TWA: 50 ppm TWA: 206 mg/m3 Skin | TWA: 50 ppm TWA: 200 mg/m3 | TWA: 50 ppm Skin |

Legend

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

NIOSH IDLH:  The National Institute for Occupational Safety and Health Immediately Dangerous to Life or Health

| Engineering Measures Personal Protective Equipment |  | Use only under a chemical fume hood. Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations and safety showers are close to the workstation location. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Eye/face Protection |  | Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. |  |  |  |
| Skin and body protection |  | Wear appropriate protective gloves and clothing to prevent skin exposure. |  |  |  |
| Respiratory Protection |  | Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149. Use a NIOSH/MSHA or European Standard EN 149 approved respirator if exposure limits are exceeded or if irritation or other symptoms are experienced. |  |  |  |
| Hygiene Measures |  | Handle in accordance with good industrial hygiene and safety practice. |  |  |  |
| 9. Physical and chemical properties |  |  |  |  |  |
| Physical State |  |  | Liquid |  |  |
| Appearance |  |  | Clear |  |  |
| Odor |  |  | Strong |  |  |
| Odor Threshold |  |  | No information available |  |  |
| pH |  |  | 6.5 |  |  |
| Melting Point/Range |  |  | 23 °C / 73.4 °F |  |  |
| Boiling Point/Range |  |  | 161 °C / 321.8 °F @ 760 mmHg |  |  |
| Flash Point |  |  | 67 °C / 152.6 °F |  |  |
| Evaporation Rate |  |  | No information available |  |  |
| Flammability (solid,gas) |  |  | No information available |  |  |
| Flammability or explosive limits Upper |  |  | No data available |  |  |
| Lower |  |  | 1.2 vol % |  |  |
| Vapor Pressure |  |  | No information available |  |  |
| Vapor Density |  |  | No information available |  |  |
| Relative Density |  |  | 0.960 |  |  |
| Solubility |  |  | No information available |  |  |
| Partition coefficient; n-octanol/water |  |  | No data available |  |  |
| Autoignition Temperature |  |  | 300 °C / 572 °F |  |  |
| Decomposition temperature |  |  | No information available |  |  |
| Viscosity |  |  | No information available |  |  |
| Molecular Formula |  |  | C6 H12 O |  |  |
| Molecular Weight |  |  | 100.16 |  |  |
|  |  |  | 10. Stability and reactivity |  |  |
| Reactive Hazard |  |  | None known, based on information available |  |
| Stability |  |  | Hygroscopic. |  |
| Conditions to Avoid |  |  | Incompatible products. Exposure to moisture. Keep away from open flames, hot surfaces and sources of ignition. |  |
| Incompatible Materials |  |  | Strong oxidizing agents, Strong acids, Strong bases |  |

Hazardous Decomposition ProductsCarbon monoxide (CO), Carbon dioxide (CO2)

| Hazardous Polymerization | Hazardous polymerization does not occur. |
| --- | --- |
| Hazardous Reactions | None under normal processing. |

# 11. Toxicological information

## Acute Toxicity

| Product Information Component Information | No acute toxicity information is available for this product |
| --- | --- |
| Toxicologically Synergistic | No information available |

Products

## Delayed and immediate effects as well as chronic effects from short and long-term exposure

| Irritation |  | Irritating to eyes, respiratory system and skin |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sensitization |  | No information available |  |  |  |  |
| Carcinogenicity |  | The table below indicates whether each agency has listed any ingredient as a carcinogen. |  |  |  |  |
| Component | CAS-No | IARC | NTP | ACGIH | OSHA | Mexico |
| Cyclohexanol | 108-93-0 | Not listed | Not listed | Not listed | Not listed | Not listed |

Mutagenic Effects	Not mutagenic in AMES Test Reproductive Effects	No information available.

Developmental Effects	No information available. Teratogenicity	No information available.

### STOT - single exposure	Respiratory system STOT - repeated exposure	None known

Aspiration hazard	No information available

Symptoms  / effects,both acute and Symptoms of overexposure may be headache, dizziness, tiredness, nausea and vomiting delayed

### Endocrine Disruptor Information	No information available

Other Adverse Effects	See actual entry in RTECS for complete information.

| 12. Ecological information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ecotoxicity Do not empty into drains. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component | Freshwater Algae |  |  |  |  | Freshwater Fish |  |  |  |  |  | Microtox |  |  |  | Water Flea |  |  |
| Cyclohexanol | 29.2 mg/L EC50 = 72 h 29 mg/L EC50 = 96 h |  |  |  |  | 1100 mg/L LC50 96 h 1033 mg/L LC50 96 h 704 mg/L LC50 96 h |  |  |  |  |  | EC50 = 42.5 mg/L 10 min EC50 = 83 mg/L 5 min EC50 = 955 mg/L 17 h |  |  |  | 578 mg/L EC50 = 24 h 500 mg/L EC50 > 24 h |  |  |
| Persistence and Degradability No information available Bioaccumulation/ Accumulation No information available. Mobility . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component |  |  |  |  |  |  |  |  | log Pow |  |  |  |  |  |  |  |  |  |
| Cyclohexanol |  |  |  |  |  |  |  |  | 1.25 |  |  |  |  |  |  |  |  |  |
| 13. Disposal considerations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Waste Disposal Methods |  |  |  | Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. Chemical waste generators must also consult local, regional, and national hazardous waste regulations to ensure complete and accurate classification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14. Transport information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DOT |  |  | Not regulated |  |  |  |  |
| TDG |  |  | Not regulated |  |  |  |  |
| IATA |  |  | Not regulated |  |  |  |  |
| IMDG/IMO |  |  | Not regulated |  |  |  |  |
|  |  |  |  | 15. Regulatory information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| International Inventories |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component |  |  | TSCA | DSL | NDSL |  |  | EINECS |  | ELINCS | NLP |  | PICCS | ENCS | AICS |  | IECSC | KECL |
| Cyclohexanol |  |  | X | X | - |  |  | 203-630-6 |  | - |  |  | X | X | X |  | X | X |

Legend:

X - Listed

- - Indicates a substance that is the subject of a Section 5(e) Consent order under TSCA.

- - Indicates a substance that is the subject of a Section 5(f) Rule under TSCA.

N - Indicates a polymeric substance containing no free-radical initiator in its inventory name but is considered to cover the designated polymer made with any free-radical initiator regardless of the amount used.

P - Indicates a commenced PMN substance

R - Indicates a substance that is the subject of a Section 6 risk management rule under TSCA. S - Indicates a substance that is identified in a proposed or final Significant New Use Rule T - Indicates a substance that is the subject of a Section 4 test rule under TSCA.

XU - Indicates a substance exempt from reporting under the Inventory Update Rule, i.e. Partial Updating of the TSCA Inventory Data Base Production and Site Reports (40 CFR 710(B).

Y1 - Indicates an exempt polymer that has a number-average molecular weight of 1,000 or greater.

Y2 - Indicates an exempt polymer that is a polyester and is made only from reactants included in a specified list of low concern reactants that comprises one of the eligibility criteria for the exemption rule.

## U.S. Federal Regulations

### TSCA 12(b)

| Component |  |  |  |  |  |  | TSCA 12(b) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cyclohexanol |  |  |  |  |  |  | Section 4 |  |  |  |
| SARA 313 |  |  |  |  |  |  |  |  |  |  |
| Component |  |  |  | CAS-No |  |  | Weight % |  | SARA 313 - Threshold Values % |  |
| Cyclohexanol |  |  |  | 108-93-0 |  |  | 98 |  | 1.0 |  |
| SARA 311/312 Hazardous Categorization Acute Health Hazard |  |  |  | Yes |  |  |  |  |  |  |
| Chronic Health Hazard |  |  |  | No |  |  |  |  |  |  |
| Fire Hazard |  |  |  | Yes |  |  |  |  |  |  |
| Sudden Release of Pressure Hazard |  |  |  | No |  |  |  |  |  |  |
| Reactive Hazard |  |  |  | No |  |  |  |  |  |  |
| Clean Water Act | Not applicable |  |  |  |  |  |  |  |  |  |
| Clean Air Act | Not applicable |  |  |  |  |  |  |  |  |  |
| OSHA Occupational Safety and Health Administration Not applicable CERCLA Not applicable California Proposition 65 This pr State Right-to-Know |  |  |  | oduct does not contain any Proposition 65 chemicals |  |  |  |  |  |  |
| Component | Massachusetts | New Jersey |  |  |  | Pennsylvania |  | Illinois |  | Rhode Island |
| Cyclohexanol | X | X |  |  |  | X |  | - |  | X |

### U.S. Department of Transportation

Reportable Quantity (RQ):	N

DOT Marine Pollutant	N DOT Severe Marine Pollutant	N

U.S. Department of Homeland Security

This product does not contain any DHS chemicals.

## Other International Regulations

Mexico - Grade	No information available

### Canada This product has been classified in accordance with the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all the information required by the CPR

WHMIS Hazard Class	B3   Combustible liquid

D1B  Toxic materials

D2B  Toxic materials

# 16. Other information

| Prepared By | Regulatory Affairs Thermo Fisher Scientific Email: EMSDS.RA@thermofisher.com |
| --- | --- |
| Creation Date | 18-Jul-2014 |
| Revision Date | 18-Jul-2014 |
| Print Date | 18-Jul-2014 |
| Revision Summary | This document has been updated to comply with the US OSHA HazCom 2012 Standard replacing the current legislation under 29 CFR 1910.1200 to align with the Globally Harmonized System of Classification and Labeling of Chemicals (GHS) |

## Disclaimer

The information provided on this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication.  The information given is designed only as a guide for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered as a warranty or quality specification.  The information relates only to the specific material designated and may not be valid for such material used in combination with any other material or in any process, unless specified in the text.

# End of SDS

CYCLOHEXANOL

|  | Warning. |
| --- | --- |

Causes serious eye irritation

Causes skin irritation

Combustible liquid

Harmful if inhaled

Harmful if swallowed

May cause respiratory irritation

Keep away from heat,  hot surfaces,  sparks,  open flames and other ignition sources. No smoking Keep cool

Avoid breathing dust/fume/gas/mist/vapours/spray

Wash ... thoroughly after handling

Do not eat,  drink or smoke when using this product

Use only outdoors or with adequate ventilation

Wear protective gloves/protective clothing/eye protection/face protection

IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell

IF ON SKIN: Wash with plenty of water

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing IF IN EYES: Rinse cautiously with water for several minutes,  Remove contact lenses,  if present and easy to do. Continue rinsing

Call a POISON CENTER or doctor/physician

Rinse mouth

If skin irritation occurs: Get medical advice/attention

If eye irritation persists: Get medical advice/attention

Take off contaminated clothing,  And wash it before reuse

In case of fire: Use ... for extinction

Store in a well-ventilated place,  Keep container tightly closed

Store locked up

Dispose of contents/container to ...

Acros Organics

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.

SAFETY DATA SHEET

Creation Date  25-Aug-2010	Revision Date  23-Oct-2014	Revision Number  1

# 1. Identification

Product Name	Cyclohexanone

Cat No. :	AC406090000, AC406090010, AC406090025, AC406092500

Synonyms	Ketohexamethylene; Pimelic ketone.

Recommended Use	Laboratory chemicals.

Uses advised against	No Information available

## Details of the supplier of the safety data sheet

| Company Fisher Scientific One Reagent Lane Fair Lawn, NJ 07410 Tel: (201) 796-7100 | Entity / Business Name Acros Organics One Reagent Lane Fair Lawn, NJ 07410 | Emergency Telephone Number For information US call: 001-800-ACROS-01 / Europe call: +32 14 57 52 11 Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99 CHEMTREC Tel. No.US:001-800-424-9300 / Europe:001-703-527-3887 |
| --- | --- | --- |

# 2. Hazard(s) identification

## Classification

This chemical is considered hazardous by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200)

Flammable liquids	Category 3

Acute oral toxicity	Category 4

Acute dermal toxicity	Category 3

Acute Inhalation Toxicity - Vapors	Category 4

Skin Corrosion/irritation	Category 2

Serious Eye Damage/Eye Irritation	Category 1 Specific target organ toxicity (single exposure)	Category 3 Target Organs -  Respiratory system, Central nervous system (CNS). Specific target organ toxicity - (repeated exposure)	Category 2 Target Organs -  Liver, Kidney.

## Label Elements

Signal Word Danger

Hazard Statements

Flammable liquid and vapor

Harmful if swallowed

Toxic in contact with skin

Harmful if inhaled

Causes skin irritation

Causes serious eye damage

May cause respiratory irritation

May cause drowsiness or dizziness

May cause damage to organs through prolonged or repeated exposure

Precautionary Statements

Prevention

Wash face, hands and any exposed skin thoroughly after handling

Do not eat, drink or smoke when using this product

Wear protective gloves/protective clothing/eye protection/face protection

Use only outdoors or in a well-ventilated area

Do not breathe dust/fume/gas/mist/vapors/spray

Keep away from heat/sparks/open flames/hot surfaces. - No smoking

Keep container tightly closed

Ground/bond container and receiving equipment

Use explosion-proof electrical/ventilating/lighting/equipment

Use only non-sparking tools

Take precautionary measures against static discharge

Keep cool

Response

Get medical attention/advice if you feel unwell

Inhalation

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Call a POISON CENTER or doctor/physician if you feel unwell

Skin

Call a POISON CENTER or doctor/physician if you feel unwell

If skin irritation occurs: Get medical advice/attention

IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower Wash contaminated clothing before reuse Eyes

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing Immediately call a POISON CENTER or doctor/physician

Ingestion

IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell Rinse mouth

Fire

In case of fire: Use CO2, dry chemical, or foam for extinction

Storage

Store in a well-ventilated place. Keep container tightly closed

Store locked up

Disposal

Dispose of contents/container to an approved waste disposal plant

Hazards not otherwise classified (HNOC)

May form explosive peroxides

# 3. Composition / information on ingredients

| Component |  |  | CAS-No | Weight % |  |
| --- | --- | --- | --- | --- | --- |
| Cyclohexanone |  |  | 108-94-1 | >95 |  |
|  |  | 4. First-aid measures |  |  |  |
| Eye Contact |  | Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required. |  |  |  |
| Skin Contact |  | Wash off immediately with plenty of water for at least 15 minutes. Immediate medical attention is required. |  |  |  |
| Inhalation |  | Move to fresh air. If breathing is difficult, give oxygen. Do not use mouth-to-mouth resuscitation if victim ingested or inhaled the substance; induce artificial respiration with a respiratory medical device. Immediate medical attention is required. |  |  |  |
| Ingestion |  | Do not induce vomiting. Call a physician or Poison Control Center immediately. |  |  |  |
| Most important symptoms/effects |  | Breathing difficulties. Causes eye burns. Symptoms of overexposure may be headache, dizziness, tiredness, nausea and vomiting |  |  |  |
| Notes to Physician |  | Treat symptomatically |  |  |  |
|  |  | 5. Fire-fighting measures |  |  |  |
| Suitable Extinguishing Media |  | CO 2, dry chemical, dry sand, alcohol-resistant foam. Cool closed containers exposed to fire with water spray. |  |  |  |
| Unsuitable Extinguishing Media |  | Water may be ineffective |  |  |  |
| Flash Point |  | 43 °C / 109 °F |  |  |  |
| Method - |  | Closed cup |  |  |  |
| Autoignition Temperature Explosion Limits |  | 520 °C / 968 °F |  |  |  |
| Upper |  | 9.4 vol % |  |  |  |
| Lower |  | 1.10 vol % |  |  |  |

Sensitivity to Mechanical Impact No information available

Sensitivity to Static Discharge	No information available

Specific Hazards Arising from the Chemical

Flammable. Containers may explode when heated. Vapors may form explosive mixtures with air. Vapors may travel to source of ignition and flash back.

Hazardous Combustion Products

Carbon monoxide (CO) Carbon dioxide (CO2)

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear. Thermal decomposition can lead to release of irritating gases and vapors.

## NFPA

| Health 1 |  | Flammability Instability 2 0 | Physical hazards N/A |  |
| --- | --- | --- | --- | --- |
|  |  | 6. Accidental release measures |  |  |
| Personal Precautions | Use personal protective equipment. Remove all sources of ignition. Take precautionary measures against static discharges. Avoid contact with skin, eyes and inhalation of vapors. |  |  |
| Environmental Precautions | Avoid release to the environment. See Section 12 for additional ecological information. |  |  |

Methods for Containment and Clean Remove all sources of ignition. Soak up with inert absorbent material. Keep in suitable,

Up	closed containers for disposal. Use spark-proof tools and explosion-proof equipment.

|  | 7. Handling and storage |
| --- | --- |
| Handling | Wear personal protective equipment. Do not get in eyes, on skin, or on clothing. Do not breathe vapors or spray mist. Do not ingest. Keep away from open flames, hot surfaces and sources of ignition. Use only non-sparking tools. Use explosion-proof equipment. Take precautionary measures against static discharges. If peroxide formation is suspected, do not open or move container. |
| Storage | Keep containers tightly closed in a dry, cool and well-ventilated place. Keep away from heat and sources of ignition. May form explosive peroxides on prolonged storage. Should crystals form in a peroxidizable liquid, peroxidation may have occurred and the product should be considered extremely dangerous. In this instance, the container should only be opened remotely by professionals. Flammables area. |
|  | 8. Exposure controls / personal protection |

## Exposure Guidelines

| Component | ACGIH TLV | OSHA PEL | NIOSH IDLH |
| --- | --- | --- | --- |
| Cyclohexanone | TWA: 20 ppm STEL: 50 ppm Skin | (Vacated) TWA: 25 ppm (Vacated) TWA: 100 mg/m3 Skin TWA: 50 ppm TWA: 200 mg/m3 | IDLH: 700 ppm TWA: 25 ppm TWA: 100 mg/m3 |
| Component | Quebec | Mexico OEL (TWA) | Ontario TWAEV |
| Cyclohexanone | TWA: 25 ppm TWA: 100 mg/m3 Skin | TWA: 50 ppm TWA: 200 mg/m3 STEL: 100 ppm STEL: 400 mg/m3 | TWA: 20 ppm STEL: 50 ppm Skin |

Legend

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

NIOSH IDLH:  The National Institute for Occupational Safety and Health Immediately Dangerous to Life or Health

| Engineering Measures Personal Protective Equipment | Ensure adequate ventilation, especially in confined areas. Use explosion-proof electrical/ventilating/lighting/equipment. Ensure that eyewash stations and safety showers are close to the workstation location. |
| --- | --- |
| Eye/face Protection | Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. |
| Skin and body protection | Wear appropriate protective gloves and clothing to prevent skin exposure. |
| Respiratory Protection | Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149. Use a NIOSH/MSHA or European Standard EN 149 approved respirator if exposure limits are exceeded or if irritation or other symptoms are experienced. |
| Hygiene Measures | Handle in accordance with good industrial hygiene and safety practice. |

# 9. Physical and chemical properties

Physical State	Liquid

| Appearance | Colorless |
| --- | --- |
| Odor | mint-like |
| Odor Threshold | 0.12 ppm |
| pH | Not applicable |
| Melting Point/Range | -47 °C / -52.6 °F |
| Boiling Point/Range | 155 °C / 311 °F @ 760 mmHg |
| Flash Point | 43 °C / 109 °F |
| Method - | Closed cup |

Evaporation Rate	No information available

Flammability (solid,gas)	Not applicable

Flammability or explosive limits

Upper	9.4 vol %

Lower	1.10 vol %

Vapor Pressure	4.5 mbar @ 20 °C

Vapor Density	3.4

Relative Density	0.947

Solubility	No information available

Partition coefficient; n-octanol/water	No data available

Autoignition Temperature	 520  °C  /  968  °F

Decomposition Temperature	No information available

Viscosity	2.2  mPas @ 20°C

Molecular Formula	C6 H10 O Molecular Weight	98.14

# 10. Stability and reactivity

Reactive Hazard	None known, based on information available

Stability	Stable under normal conditions. May form explosive peroxides.

Conditions to Avoid	Incompatible products. Heat, flames and sparks. The presence of oxygen or prolonged standing in or exposure to direct sunlight may lead to formation of unstable peroxides, which may explode spontaneously or when heated.

Incompatible Materials	Strong oxidizing agents, Strong acids Hazardous Decomposition ProductsCarbon monoxide (CO), Carbon dioxide (CO2)

Hazardous Polymerization	Hazardous polymerization does not occur.

Hazardous Reactions	None under normal processing.

|  | 11. Toxicological information |  |  |
| --- | --- | --- | --- |
| Acute Toxicity Product Information Component Information |  |  |  |
| Component | LD50 Oral | LD50 Dermal | LC50 Inhalation |
| Cyclohexanone | 1544 mg/kg ( Rat ) | 947 mg/kg ( Rabbit ) | 8000 ppm ( Rat ) 4 h |

Toxicologically Synergistic	No information available

Products

## Delayed and immediate effects as well as chronic effects from short and long-term exposure

| Irritation |  |  | Causes eye burns; Irritating to skin |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sensitization |  |  | No information available |  |  |  |  |  |  |
| Carcinogenicity |  |  | The table below indicates whether each agency has listed any ingredient as a carcinogen. |  |  |  |  |  |  |
| Component | CAS-No |  | IARC | NTP | ACGIH | OSHA | Mexico |  |  |
| Cyclohexanone | 108-94-1 |  | group 3 | Not listed | A3 | Not listed | Not listed |  |  |
| Mutagenic Effects |  |  | No information available |  |  |  |  |  |
| Reproductive Effects |  |  | Experiments have shown reproductive toxicity effects on laboratory animals. |  |  |  |  |  |
| Developmental Effects |  |  | Developmental effects have occurred in experimental animals. |  |  |  |  |  |
| Teratogenicity |  |  | Teratogenic effects have occurred in experimental animals. |  |  |  |  |  |

STOT - single exposure	Respiratory system Central nervous system (CNS) STOT - repeated exposure	Liver Kidney

Aspiration hazard	No information available

Symptoms  / effects,both acute and Symptoms of overexposure may be headache, dizziness, tiredness, nausea and vomiting delayed

Endocrine Disruptor Information	No information available

Other Adverse Effects	See actual entry in RTECS for complete information.

|  | 12. Ecological information |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ecotoxicity Do not empty into drains. |  |  |  |  |  |  |  |  |
| Component | Freshwater Algae |  | Freshwater Fish |  | Microtox |  |  | Water Flea |
| Cyclohexanone | Not listed |  | Leusiscus idus: LC50>500mg/L 48h |  | EC50 = 18.5 mg/L 5 min EC50 = 21.3 mg/L 10 min EC50 = 25 mg/L 5 min |  |  | Not listed |
| Persistence and Degradability Insoluble in water Persistence is unlikely based on information available. Bioaccumulation/ Accumulation No information available. Mobility Is not likely mobile in the environment due its low water solubility. |  |  |  |  |  |  |  |  |
| Component |  |  |  | log Pow |  |  |  |  |
| Cyclohexanone |  |  |  | 0.86 |  |  |  |  |
| 13. Disposal considerations |  |  |  |  |  |  |  |  |
| Waste Disposal Methods |  | Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. Chemical waste generators must also consult local, regional, and national hazardous waste regulations to ensure complete and accurate classification. |  |  |  |  |  |  |
| Component |  |  | RCRA - U Series Wastes |  |  | RCRA - P Series Wastes |  |  |
| Cyclohexanone - 108-94-1 |  |  | U057 |  |  | - |  |  |

# 14. Transport information

| DOT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UN-No |  |  | UN1915 |  |  |  |
| Proper Shipping Name |  |  | CYCLOHEXANONE |  |  |  |
| Hazard Class |  |  | 3 |  |  |  |
| Packing Group TDG |  |  | III |  |  |  |
| UN-No |  |  | UN1915 |  |  |  |
| Proper Shipping Name |  |  | CYCLOHEXANONE |  |  |  |
| Hazard Class |  |  | 3 |  |  |  |
| Packing Group IATA |  |  | III |  |  |  |
| UN-No |  |  | UN1915 |  |  |  |
| Proper Shipping Name |  |  | CYCLOHEXANONE |  |  |  |
| Hazard Class |  |  | 3 |  |  |  |
| Packing Group IMDG/IMO |  |  | III |  |  |  |
| UN-No |  |  | UN1915 |  |  |  |
| Proper Shipping Name |  |  | CYCLOHEXANONE |  |  |  |
| Hazard Class |  |  | 3 |  |  |  |
| Packing Group |  |  | III |  |  |  |
|  |  |  | 15. Regulatory information |  |  |  |  |  |  |  |  |  |  |
| International Inventories |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component | TSCA |  | DSL | NDSL | EINECS |  | ELINCS | NLP | PICCS | ENCS | AICS | IECSC | KECL |
| Cyclohexanone | X |  | X | - | 203-631-1 |  | - |  | X | X | X | X | X |

Legend:

X - Listed

- - Indicates a substance that is the subject of a Section 5(e) Consent order under TSCA.

- - Indicates a substance that is the subject of a Section 5(f) Rule under TSCA.

N - Indicates a polymeric substance containing no free-radical initiator in its inventory name but is considered to cover the designated polymer made with any free-radical initiator regardless of the amount used.

P - Indicates a commenced PMN substance

R - Indicates a substance that is the subject of a Section 6 risk management rule under TSCA. S - Indicates a substance that is identified in a proposed or final Significant New Use Rule T - Indicates a substance that is the subject of a Section 4 test rule under TSCA.

XU - Indicates a substance exempt from reporting under the Inventory Update Rule, i.e. Partial Updating of the TSCA Inventory Data Base Production and Site Reports (40 CFR 710(B).

Y1 - Indicates an exempt polymer that has a number-average molecular weight of 1,000 or greater.

Y2 - Indicates an exempt polymer that is a polyester and is made only from reactants included in a specified list of low concern reactants that comprises one of the eligibility criteria for the exemption rule.

## U.S. Federal Regulations

| TSCA 12(b) Not applicable SARA 313 Not applicable SARA 311/312 Hazardous Categorization |  |
| --- | --- |
| Acute Health Hazard | Yes |
| Chronic Health Hazard | No |
| Fire Hazard | Yes |
| Sudden Release of Pressure Hazard | No |
| Reactive Hazard Clean Water Act Not applicable Clean Air Act Not applicable | No |

OSHA Occupational Safety and Health Administration Not applicable

CERCLA

This material, as supplied, contains one or more substances regulated as a hazardous substance under the Comprehensive

Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302)

| Component |  |  | Hazardous Substances RQs |  |  | CERCLA EHS RQs |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cyclohexanone |  |  | 5000 lb |  |  | - |  |
| California Proposition 65 This product does not contain any Proposition 65 ch State Right-to-Know |  |  |  |  |  | emicals |  |
| Component | Massachusetts | New Jersey |  | Pennsylvania |  | Illinois | Rhode Island |
| Cyclohexanone | X | X |  | X |  | X | X |

U.S. Department of Transportation

| Reportable Quantity (RQ): | N |
| --- | --- |
| DOT Marine Pollutant | N |
| DOT Severe Marine Pollutant | N |

U.S. Department of Homeland Security

This product does not contain any DHS chemicals.

## Other International Regulations

Mexico - Grade	No information available

Canada

This product has been classified in accordance with the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all the information required by the CPR

WHMIS Hazard Class	B3   Combustible liquid

D1B  Toxic materials

D2B  Toxic materials

# 16. Other information

| Prepared By | Regulatory Affairs Thermo Fisher Scientific Email: EMSDS.RA@thermofisher.com |
| --- | --- |
| Creation Date | 25-Aug-2010 |
| Revision Date | 23-Oct-2014 |
| Print Date | 23-Oct-2014 |
| Revision Summary | This document has been updated to comply with the US OSHA HazCom 2012 Standard replacing the current legislation under 29 CFR 1910.1200 to align with the Globally Harmonized System of Classification and Labeling of Chemicals (GHS) |

Disclaimer

The information provided on this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication.  The information given is designed only as a guide for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered as a warranty or quality specification.  The information relates only to the specific material designated and may not be valid for such material used in combination with any other material or in any process, unless specified in the text.

# End of SDS

CYCLOHEXANONE

|  | Danger. |
| --- | --- |

Causes serious eye damage

Causes skin irritation

Flammable liquid and vapour

Harmful if inhaled

Harmful if swallowed

May cause damage to organs through prolonged or repeated exposure

May cause drowsiness or dizziness

May cause respiratory irritation

Toxic in contact with skin

Keep away from heat,  hot surfaces,  sparks,  open flames and other ignition sources. No smoking

Keep container tightly closed

Keep cool

Ground and bond container and receiving equipment

Use explosion-proof electrical/ventilating/lighting/.../equipment

Use only non-sparking tools

Take action to prevent static discharges

Do not breathe dust/fume/gas/mist/vapours/spray

Wash ... thoroughly after handling

Do not eat,  drink or smoke when using this product

Use only outdoors or with adequate ventilation

Wear protective gloves/protective clothing/eye protection/face protection

IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell

IF ON SKIN (or hair): Take off immediately all contaminated clothing,  Rinse skin with water/shower

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Acros OrganicsIF IN EYES: Rinse cautiously with water for several minutes,  Remove contact lenses,  if present

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.

and easy to do. Continue rinsing

Immediately call a POISON CENTER/doctor/…

Call a POISON CENTER or doctor/physician

Rinse mouth

If skin irritation occurs: Get medical advice/attention

Wash contaminated clothing before reuse

In case of fire: Use ... for extinction

Store in a well-ventilated place,  Keep container tightly closed

Store locked up

Dispose of contents/container to ...

Acros Organics

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.

SAFETY DATA SHEET

| Creation Date 18-Jun-2009 | Revision Date 28-Apr-2016 | Revision Number 2 |
| --- | --- | --- |

# 1. Identification

| Product Name Water Cat No. : AC389390000; AC389390010; AC389390025 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Synonyms No information available Recommended Use Laboratory chemicals. Uses advised against No Information available Details of the supplier of the safety data sheet |  |  |  |  |  |  |  |  |
| Company Entity / Business Name Fisher Scientific Acros Organics One Reagent Lane One Reagent Lane Fair Lawn, NJ 07410 Fair Lawn, NJ 07410 Tel: (201) 796-7100 |  |  |  |  |  | Emergency Telephone Number For information US call: 001-800-ACROS-01 / Europe call: +32 14 57 52 11 Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99 CHEMTREC Tel. No.US:001-800-424-9300 / Europe:001-703-527-3887 |  |  |
|  |  | 2. Hazard(s) identification |  |  |  |  |  |  |
| Classification Classification under 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200) |  |  |  |  |  |  |  |  |
| Based on available data, the classification criteria are not met |  |  |  |  |  |  |  |  |
| Label Elements None required Hazards not otherwise classified (HNOC) None identified |  |  |  |  |  |  |  |  |
| 3. Composition / information on ingredients |  |  |  |  |  |  |  |  |
| Component |  |  | CAS-No |  |  |  | Weight % |  |
| Water |  |  | 7732-18-5 |  |  |  | 100 |  |
| 4. First-aid measures |  |  |  |  |  |  |  |  |
| Eye Contact |  | Immediate medical attention is not required. |  |  |
| Skin Contact |  | Immediate medical attention is not required. |  |  |
| Inhalation |  | Immediate medical attention is not required. |  |  |
| Ingestion |  | Immediate medical attention is not required. |  |  |
| Most important symptoms/effects |  | No information available. |  |  |
| Notes to Physician |  | Treat symptomatically |  |  |

# 5. Fire-fighting measures

| Suitable Extinguishing Media | Substance is nonflammable; use agent most appropriate to extinguish surrounding fire. |
| --- | --- |
| Unsuitable Extinguishing Media | No information available |
| Flash Point | Not applicable |
| Method - | No information available |
| Autoignition Temperature Explosion Limits | No information available |
| Upper | No data available |
| Lower | No data available |

Sensitivity to Mechanical Impact No information available

Sensitivity to Static Discharge	No information available

Specific Hazards Arising from the Chemical None known.

Hazardous Combustion Products

None

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

## NFPA

Health	Flammability	Instability	Physical hazards

0	0	0	N/A

|  |  | 6. Accidental release measures |
| --- | --- | --- |
| Personal Precautions |  | No special precautions required. If spilled, take caution, as material can cause surfaces to become very slippery. |
| Environmental Precautions |  | No special environmental precautions required. See Section 12 for additional ecological information. |
| Methods for Containment an Up | d Clean Soak up with inert absorbent material. |  |
|  | 7. Handling and storage |  |
| Handling | No special handling advice required. |  |
| Storage | No special storage conditions required. |  |
|  | 8. Exposure controls / personal protection |  |

Exposure Guidelines 	This product does not contain any known or suspected reproductive hazards

Engineering Measures	None under normal use conditions.

## Personal Protective Equipment

Eye/face Protection	Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166.

| Skin and body protection |  | No special protective equipment required. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Respiratory Protection |  | No protective equipment is needed under normal use conditions. |  |  |  |
| Hygiene Measures |  | Handle in accordance with good industrial hygiene and safety practice. |  |  |  |
|  |  | 9. Physical and chemical properties |  |  |  |  |  |
| Physical State |  |  |  | Liquid |  |  |  |
| Appearance |  |  |  | Clear, Colorless |  |  |  |
| Odor |  |  |  | Odorless |  |  |  |
| Odor Threshold |  |  |  | No information available |  |  |  |
| pH |  |  |  | No information available |  |  |  |
| Melting Point/Range |  |  |  | No data available |  |  |  |
| Boiling Point/Range |  |  |  | 100 °C / 212 °F |  |  |  |
| Flash Point |  |  |  | Not applicable |  |  |  |
| Evaporation Rate |  |  |  | No information available |  |  |  |
| Flammability (solid,gas) |  |  |  | Not applicable |  |  |  |
| Flammability or explosive limits Upper |  |  |  | No data available |  |  |  |
| Lower |  |  |  | No data available |  |  |  |
| Vapor Pressure |  |  |  | 17.5 mmHg @ 20 °C |  |  |  |
| Vapor Density |  |  |  | No information available |  |  |  |
| Specific Gravity |  |  |  | 1.000 |  |  |  |
| Solubility |  |  |  | No information available |  |  |  |
| Partition coefficient; n-octanol/water |  |  |  | No data available |  |  |  |
| Autoignition Temperature |  |  |  | No information available |  |  |  |
| Decomposition Temperature |  |  |  | No information available |  |  |  |
| Viscosity |  |  |  | No information available |  |  |  |
| Molecular Formula |  |  |  | H2O |  |  |  |
| Molecular Weight |  |  |  | 18.02 |  |  |  |
|  |  |  |  | 10. Stability and reactivity |  |  |  |
| Reactive Hazard |  |  |  | None known, based on information available |  |  |  |
| Stability |  |  |  | Stable under normal conditions. |  |  |  |
| Conditions to Avoid |  |  |  | None known. |  |  |  |
| Incompatible Materials None known Hazardous Decomposition ProductsNone Hazardous Polymerization Hazardous polymerization does not occur. |  |  |  |  |  |  |  |
| Hazardous Reactions |  | None under normal processing. |  |  |  |  |  |
|  |  | 11. Toxicological information |  |  |  |  |  |
| Acute Toxicity Product Information Component Information |  | No acute toxicity information is available for this product |  |  |  |  |  |
| Component |  | LD50 Oral |  |  | LD50 Dermal | LC50 Inhalation |  |
| Water |  | - |  |  | Not listed | Not listed |  |

Toxicologically Synergistic	No information available

Products

## Delayed and immediate effects as well as chronic effects from short and long-term exposure

| Irritation |  |  | No information available |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sensitization |  |  | No information available |  |  |  |  |  |
| Carcinogenicity |  |  | The table below indicates whether each agency has listed any ingredient as a carcinogen. |  |  |  |  |  |
| Component | CAS-No |  | IARC | NTP | ACGIH | OSHA | Mexico |  |
| Water | 7732-18-5 |  | Not listed | Not listed | Not listed | Not listed | Not listed |  |
| Mutagenic Effects |  |  | No information available |  |  |  |  |  |
| Reproductive Effects |  |  | No information available. |  |  |  |  |  |
| Developmental Effects |  |  | No information available. |  |  |  |  |  |
| Teratogenicity |  |  | No information available. |  |  |  |  |  |
| STOT - single exposure |  |  | None known |  |  |  |  |  |
| STOT - repeated exposure |  |  | None known |  |  |  |  |  |
| Aspiration hazard |  |  | No information available |  |  |  |  |  |
| Symptoms / effects,both acute and delayed |  |  | No information available |  |  |  |  |  |
| Endocrine Disruptor Information |  |  | No information available |  |  |  |  |  |
| Other Adverse Effects |  |  | The toxicological properties have not been fully investigated. |  |  |  |  |  |
|  |  |  | 12. Ecological information |  |  |  |  |  |

## Ecotoxicity

Contains no substances known to be hazardous to the environment or that are not degradable in waste water treatment plants.

Persistence and Degradability	No information available Bioaccumulation/ Accumulation	No information available.

Mobility	.

|  |  | 13. Disposal considerations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Waste Disposal Methods |  | Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. Chemical waste generators must also consult local, regional, and national hazardous waste regulations to ensure complete and accurate classification. |  |  |  |  |  |  |  |  |  |
|  |  | 14. Transport information |  |  |  |  |  |  |  |  |  |
| DOT |  | Not regulated |  |  |  |  |  |  |  |  |  |
| TDG |  | Not regulated |  |  |  |  |  |  |  |  |  |
| IATA |  | Not regulated |  |  |  |  |  |  |  |  |  |
| IMDG/IMO |  | Not regulated |  |  |  |  |  |  |  |  |  |
|  |  | 15. Regulatory information |  |  |  |  |  |  |  |  |  |
| International Inventories |  |  |  |  |  |  |  |  |  |  |  |
| Component | TSCA | DSL | NDSL | EINECS | ELINCS | NLP | PICCS | ENCS | AICS | IECSC | KECL |
| Water | X | X | - | 231-791-2 | - |  | X | - | X | X | X |

Legend:

X - Listed

E - Indicates a substance that is the subject of a Section 5(e) Consent order under TSCA. F - Indicates a substance that is the subject of a Section 5(f) Rule under TSCA.

N - Indicates a polymeric substance containing no free-radical initiator in its inventory name but is considered to cover the designated polymer made with any free-radical initiator regardless of the amount used.

P - Indicates a commenced PMN substance

R - Indicates a substance that is the subject of a Section 6 risk management rule under TSCA. S - Indicates a substance that is identified in a proposed or final Significant New Use Rule T - Indicates a substance that is the subject of a Section 4 test rule under TSCA.

XU - Indicates a substance exempt from reporting under the Inventory Update Rule, i.e. Partial Updating of the TSCA Inventory Data Base Production and Site Reports (40 CFR 710(B).

Y1 - Indicates an exempt polymer that has a number-average molecular weight of 1,000 or greater.

Y2 - Indicates an exempt polymer that is a polyester and is made only from reactants included in a specified list of low concern reactants that comprises one of the eligibility criteria for the exemption rule.

## U.S. Federal Regulations

TSCA 12(b)	Not applicable

SARA 313	Not applicable

| SARA 311/312 Hazard Categories |  |
| --- | --- |
| Acute Health Hazard | No |
| Chronic Health Hazard | No |
| Fire Hazard | No |
| Sudden Release of Pressure Hazard | No |
| Reactive Hazard CWA (Clean Water Act) Not applicable Clean Air Act Not applicable | No |

OSHA Occupational Safety and Health Administration Not applicable

CERCLA

Not applicable

| California Proposition 65 | This product does not contain any Proposition 65 chemicals |
| --- | --- |
| U.S. State Right-to-Know | Not applicable |

Regulations

| Component | Massachusetts | New Jersey | Pennsylvania | Illinois | Rhode Island |
| --- | --- | --- | --- | --- | --- |
| Water | - | - | X | - | - |

U.S. Department of Transportation

Reportable Quantity (RQ):	N

DOT Marine Pollutant	N DOT Severe Marine Pollutant	N

U.S. Department of Homeland Security

This product does not contain any DHS chemicals.

## Other International Regulations

Mexico - Grade	No information available

Canada

This product has been classified in accordance with the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all the information required by the CPR

WHMIS Hazard Class	Non-controlled

# 16. Other information

| Prepared By | Regulatory Affairs Thermo Fisher Scientific Email: EMSDS.RA@thermofisher.com |
| --- | --- |
| Creation Date | 18-Jun-2009 |
| Revision Date | 28-Apr-2016 |
| Print Date | 28-Apr-2016 |
| Revision Summary | This document has been updated to comply with the US OSHA HazCom 2012 Standard replacing the current legislation under 29 CFR 1910.1200 to align with the Globally Harmonized System of Classification and Labeling of Chemicals (GHS) |

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text

# End of SDS

|  | SAFETY DATA SHEET | Page : 1/10 |
| --- | --- | --- |
|  |  | Revised edition no : 7.0 |
|  |  | Revision date : 2025-07-03 |
|  |  | Supersedes version of : 2024-02-02 |
| Compressed air |  | NOAL_0000A |
|  |  | Country : DK / Language : EN |

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

## 1.1. Product identifier

Trade name 	: Compressed air, Air, Medical air, Syntetisk luft, Synthetic air N40, Synthetic air N52,

Breathing air, Alphagaz 1 Air, Technical air SDS no 	: NOAL_0000A

## 1.2. Relevant identified uses of the substance or mixture and uses advised against

Relevant identified uses 	: Industrial and professional uses. Perform risk assessment prior to use.

Test gas/Calibration gas.

Laboratory use.

Purging.

Industrial and professional use for chemical analysis, calibration, (routine) quality control, laboratory use, under controlled conditions.

Contact supplier for more information on uses.

Uses advised against 	: None.

1.3. Details of the supplier of the safety data sheet

## Company identification

| Supplier AIR LIQUIDE Denmark A/S Uraniavej 6 8700 Horsens - DENMARK T +45 76 25 25 25 |  |
| --- | --- |
| E-Mail address (competent person) | : eunordic-sds@airliquide.com |

### 1.4. Emergency telephone number

| Country | Organisation/Company | Address | Emergency number | Comment |
| --- | --- | --- | --- | --- |
| Denmark | Giftlinjen Bispebjerg Hospital | Bispebjerg Bakke 23E Opgang 20 C 2400 | +45 82 12 12 12 |  |

# SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

## Classification according to Regulation (EC) No. 1272/2008 [CLP]

Physical hazards 	Gases under pressure : Compressed gas 	H280

2.2. Label elements

## Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP) 	:

|  | GHS04 |
| --- | --- |
| Signal word (CLP) | : Warning |
| Hazard statements (CLP) | : H280 - Contains gas under pressure; may explode if heated. |
| Precautionary statements (CLP) |  |
| - Storage | : P410+P403 - Protect from sunlight. Store in a well-ventilated place. |

### 2.3. Other hazards

Mixture does not contain substance (s) classified as PBT or vPvB in concentrations above

0,1 weight %.

The substance/mixture has no endocrine disrupting properties.

The mixture does not contain components considered to have endocrine disrupting properties according to REACH Article 57(f) or Commission Delegated regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at levels of 0.1% or higher.

Mixture does not contain substance(s) classified as PMT or vPvM in concentrations above 0.1 weight %.

# SECTION 3: Composition/information on ingredients

| 3.1. Substances | Not established. |
| --- | --- |
| 3.2. Mixtures |  |

| Name | Product identifier | Composition [V%]: | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Nitrogen | CAS-No.: 7727-37-9 EC-No.: 231-783-9 EC Index-No.: --- REACH-no: *1 | 79 | Press. Gas (Comp.), H280 |
| Oxygen | CAS-No.: 7782-44-7 EC-No.: 231-956-9 EC Index-No.: 008-001-00-8 REACH-no: *1 | 21 | Ox. Gas 1, H270 Press. Gas (Comp.), H280 |

Full text of H- and EUH-statements: see section 16

Contains no other components or impurities which will influence the classification of the product.

*1: Listed in Annex IV / V REACH, exempted from registration.

*3: Registration not required: Substance manufactured or imported < 1t/y.

# SECTION 4: First aid measures

## 4.1. Description of first aid measures

- Inhalation 	: Remove victim to uncontaminated area wearing self contained breathing apparatus. Keep

victim warm and rested. Call a doctor. Perform cardiopulmonary resuscitation if breathing stopped.

- Skin contact 	: Adverse effects not expected from this product.

- Eye contact 	: Adverse effects not expected from this product.

- Ingestion 	: Ingestion is not considered a potential route of exposure.

## 4.2. Most important symptoms and effects, both acute and delayed

In high concentrations may cause asphyxiation. Symptoms may include loss of

mobility/consciousness. Victim may not be aware of asphyxiation. See section 11.

4.3. Indication of any immediate medical attention and special treatment needed

None.

# SECTION 5: Firefighting measures

## 5.1. Extinguishing media

- Suitable extinguishing media 	: Water spray or fog.

Product does not burn, use fire control measures appropriate for the surrounding fire. - Unsuitable extinguishing media 	: Do not use water jet to extinguish. 5.2. Special hazards arising from the substance or mixture

| Specific hazards | : Exposure to fire may cause containers to rupture/explode. |
| --- | --- |
| Hazardous combustion products 5.3. Advice for firefighters | : None. |
| Specific methods | : Use fire control measures appropriate for the surrounding fire. Exposure to fire and heat radiation may cause gas receptacles to rupture. Cool endangered receptacles with water spray jet from a protected position. Prevent water used in emergency cases from entering sewers and drainage systems. If possible, stop flow of product. Use water spray or fog to knock down fire fumes if possible. Move containers away from the fire area if this can be done without risk. |
| Special protective equipment for fire fighters | : In confined space use self-contained breathing apparatus. Standard protective clothing and equipment (Self Contained Breathing Apparatus) for fire fighters. Standard EN 137 - Self-contained open-circuit compressed air breathing apparatus with full face mask. Standard EN 469 - Protective clothing for firefighters. Standard - EN 659: Protective gloves for firefighters. EN 15090 Footwear for firefighters. EN 443 Helmets for fire fighting in buildings and other structures. |

# SECTION 6: Accidental release measures

## 6.1. Personal precautions, protective equipment and emergency procedures

| For non-emergency personnel | : Act in accordance with local emergency plan. Try to stop release. Evacuate area. Ensure adequate air ventilation. See section 8 of the SDS for more information on personal protective equipment |
| --- | --- |
| For emergency responders 6.2. Environmental precautions | : Wear self-contained breathing apparatus when entering area unless atmosphere is proved to be safe. Oxygen detectors should be used when asphyxiating gases may be released. See section 5.3 of the SDS for more information. |
|  | Try to stop release. |

6.3. Methods and material for containment and cleaning up

Ventilate area.

6.4. Reference to other sections

See also sections 8 and 13.

# SECTION 7: Handling and storage

## 7.1. Precautions for safe handling

| Safe use of the product | : Do not breathe gas. The product must be handled in accordance with good industrial hygiene and safety procedures. Only experienced and properly instructed persons should handle gases under pressure. Consider pressure relief device(s) in gas installations. Ensure the complete gas system was (or is regularily) checked for leaks before use. Do not smoke while handling product. Use only properly specified equipment which is suitable for this product, its supply pressure and temperature. Contact your gas supplier if in doubt. Avoid suck back of water, acid and alkalis. |
| --- | --- |
| Safe handling of the gas receptacle | : Refer to supplier's container handling instructions. Do not allow backfeed into the container. Protect containers from physical damage; do not drag, roll, slide or drop. When moving cylinders, even for short distances, use a cart (trolley, hand truck, etc.) designed to transport cylinders. Leave valve protection caps, when provided, in place until the container has been secured against either a wall or bench or placed in a container stand and is ready for use. If user experiences any difficulty operating valve discontinue use and contact supplier. Never attempt to repair or modify container valves or safety relief devices. Damaged valves should be reported immediately to the supplier. Keep container valve outlets clean and free from contaminants particularly oil and water. Replace valve outlet caps or plugs and container caps where supplied as soon as container is disconnected from equipment. Close container valve after each use and when empty, even if still connected to equipment. Never attempt to transfer gases from one cylinder/container to another. Never use direct flame or electrical heating devices to raise the pressure of a container. Do not remove or deface labels provided by the supplier for the identification of the content of the container. Suck back of water into the container must be prevented. Open valve slowly to avoid pressure shock. |

## 7.2. Conditions for safe storage, including any incompatibilities

Observe all regulations and local requirements regarding storage of containers.

Containers should not be stored in conditions likely to encourage corrosion.

Container valve guards or caps, when provided, should be in place.

Containers should be stored in the vertical position and properly secured to prevent them from falling over.

Stored containers should be periodically checked for general condition and leakage.

Keep container below 50°C in a well ventilated place.

Store containers in location free from fire risk and away from sources of heat and ignition. Keep away from combustible materials.

7.3. Specific end use(s)

None.

# SECTION 8: Exposure controls/personal protection

## 8.1. Control parameters

| OEL (Occupational Exposure Limits) | : None available. |
| --- | --- |
| DNEL (Derived-No Effect Level) | : None available. |
| PNEC (Predicted No-Effect Concentration) | : None available. |

## 8.2. Exposure controls

### 8.2.1. Appropriate engineering controls

Provide adequate general and local exhaust ventilation.

Systems under pressure  should be regularily checked for leakages.

Oxygen detectors should be used when asphyxiating gases may be released. Consider the use of a work permit system e.g. for maintenance activities.

### 8.2.2. Individual protection measures, e.g. personal protective equipment

A risk assessment should be conducted and documented in each work area to assess the

risks related to the use of the product and to select the PPE that matches the relevant risk.

The following recommendations should be considered:

PPE compliant to the recommended EN/ISO standards should be selected.

- Eye/face protection 	: Wear safety glasses with side shields.

Standard EN 166 - Personal eye-protection - specifications.

- Skin protection

  - Hand protection 	: Wear working gloves when handling gas containers.

Standard EN 388 - Protective gloves against mechanical risks, performance level 1 or higher. Recommended types include wrist gloves from leather or synthetic material with equivalent performance, fabric gloves, fabric gloves with leather palms.

  - Other 	: Wear safety shoes while handling containers.

Standard EN ISO 20345 - Personal protective equipment - Safety footwear.

- Respiratory protection 	: Standard EN 137 - Self-contained open-circuit compressed air breathing apparatus with full face mask.

When indicated by a risk assessment, Respiratory Protective Equipment must be used. The selection of the Respiratory Protective Device (RPD) must be based on known or anticipated exposure levels, the hazards of the product and the safe working limits of the selected RPD.

Self contained breathing apparatus (SCBA) or positive pressure airline with mask are to be used in oxygen-deficient atmospheres.

Self contained breathing apparatus is recommended, where unknown exposure may be expected, e.g. during maintenance activities on installation systems.

- Thermal hazards 	: None in addition to the above sections.

8.2.3. Environmental exposure controls

None necessary.

# SECTION 9: Physical and chemical properties

## 9.1. Information on basic physical and chemical properties

| Appearance |  |
| --- | --- |
| - Physical state at 20°C / 101.3kPa | : Gas |
| - Colour | : Colourless |
| Odour | : Odourless. Odour threshold is subjective and inadequate to warn of overexposure. |
| pH | : Not applicable for gases and gas mixtures. |
| Melting point / Freezing point | : Not known. |
| Boiling point | : Not known. |
| Flash point | : Not applicable for gases and gas mixtures. |
| Flammability | : Non flammable. |
| Explosive limits | : Non flammable. |
| Lower explosion limit | : Not available |
| Upper explosion limit | : Not available |
| Vapour pressure [20°C] | : Not applicable. |
| Vapour pressure [50°C] | : Not applicable. |
| Density | : Not applicable |
| Vapour density | : Not applicable for gases and gas mixtures. |
| Relative density, liquid (water=1) | : Not applicable. |
| Relative density, gas (air=1) | : 1 |

Water solubility 	: Not known, but considered to have low solubility.

Partition coefficient n-octanol/water (Log Kow) 	: Not applicable for inorganic products.

Auto-ignition temperature 	: Non flammable.

Decomposition temperature 	: Not applicable.

Viscosity, kinematic 	: No reliable data available.

Particle characteristics 	: Not applicable for gases and gas mixtures.

## 9.2. Other information

### 9.2.1. Information with regard to physical hazard classes

| Explosive properties | : Not applicable. |
| --- | --- |
| Oxidising properties | : Not applicable. |
| Critical temperature [°C] 9.2.2. Other safety characteristics | : Not known. |
| Molar mass | : 29 g/mol |
| Evaporation rate | : Not applicable for gases and gas mixtures. |

# SECTION 10: Stability and reactivity

10.1. Reactivity

No reactivity hazard other than the effects described in sub-sections below.

10.2. Chemical stability

Stable under normal conditions.

## 10.3. Possibility of hazardous reactions

|  | None. None under normal use. |
| --- | --- |
| Reactivity 10.4. Conditions to avoid | : This mixture contains components with the following reactivity : Violently oxidises organic material. |
|  | Avoid moisture in installation systems. |

10.5. Incompatible materials

For additional information on compatibility refer to ISO 11114.

10.6. Hazardous decomposition products

None.

# SECTION 11: Toxicological information

## 11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

| Acute toxicity | : No toxicological effects from this product. |
| --- | --- |
| Skin corrosion/irritation | : No known effects from this product. |
| Serious eye damage/irritation | : No known effects from this product. |
| Respiratory or skin sensitisation | : No known effects from this product. |
| Germ cell mutagenicity | : No known effects from this product. |
| Carcinogenicity | : No known effects from this product. |
| Toxic for reproduction : Fertility | : No known effects from this product. |
| Toxic for reproduction : unborn child | : No known effects from this product. |
| STOT-single exposure | : No known effects from this product. |
| STOT-repeated exposure | : No known effects from this product. |

| Aspiration hazard 11.2. Information on other hazards |  | : Not applicable for gases and gas mixtures. |  |  |
| --- | --- | --- | --- | --- |
| Other information |  | : The substance/mixture has no endocrine disrupting properties. |  |  |
| SECTION 12: Ecological information |  |  |  |  |
| 12.1. Toxicity Assessment |  | : | No ecological damage caused by this product. |  |
| EC50 48h - Daphnia magna [mg/l] |  | : | No data available. |  |
| EC50 72h - Algae [mg/l] |  | : | No data available. |  |
| LC50 96 h - Fish [mg/l] |  | : | No data available. |  |
| 12.2. Persistence and degradability Assessment |  | : | No ecological damage caused by this product. |  |
| 12.3. Bioaccumulative potential Assessment |  | : | No data available. |  |
| 12.4. Mobility in soil Assessment |  | : | Because of its high volatility, the product is unlikely to cause ground or water pollution. Partition into soil is unlikely. |  |
| 12.5. Results of PBT and vPvB assessment Assessment |  | : | No data available. |  |
| 12.6. Endocrine disrupting properties |  |  | The substance/mixture has no endocrine disrupting properties. |  |
| 12.7. Other adverse effects Other adverse effects |  | : | No known effects from this product. |  |
| Effect on the ozone layer |  | : | None. |  |
| Effect on global warming |  | : | No known effects from this product. |  |
| SECTION 13: Disposal considerations |  |  |  |  |
| 13.1. Waste treatment methods |  |  | May be vented to atmosphere. May be vented to atmosphere in a well ventilated place. Do not discharge into any place where its accumulation could be dangerous. Return unused product in original container to supplier. |  |
| List of hazardous waste codes (from Commission Decision 2000/532/EC as amended) |  | : | 16 05 05 : Gases in pressure containers other than those mentioned in 16 05 04. |  |
| 13.2. Additional information |  |  | External treatment and disposal of waste should comply with applicable local and/or national regulations. |  |
| SECTION 14: Transport information |  |  |  |  |

## 14.1. UN number or ID number

In accordance with ADR / RID / IMDG / IATA / ADN

UN-No. 	: 1002

## 14.2. UN proper shipping name

| Transport by road/rail/inland waterways (ADR/RID/ADN) | : AIR, COMPRESSED |
| --- | --- |
| Transport by air (ICAO-TI / IATA-DGR) | : Air, compressed |
| Transport by sea (IMDG) 14.3. Transport hazard class(es) | : AIR, COMPRESSED |

Labelling 	:

|  | 2.2 : Non-flammable, non-toxic gases. |
| --- | --- |
| Transport by road/rail/inland waterways (ADR/RID/ADN) |  |
| Class | : 2 |
| Classification code | : 1A |
| Hazard identification number | : 20 |
| Tunnel Restriction | : E - Passage forbidden through tunnels of category E |
| Transport by air (ICAO-TI / IATA-DGR) |  |
| Class / Div. (Sub. risk(s)) | : 2.2 |
| Transport by sea (IMDG) |  |
| Class / Div. (Sub. risk(s)) | : 2.2 |
| Emergency Schedule (EmS) - Fire | : F-C |
| Emergency Schedule (EmS) - Spillage 14.4. Packing group | : S-V |
| Transport by road/rail/inland waterways (ADR/RID/ADN) | : Not established. |
| Transport by air (ICAO-TI / IATA-DGR) | : Not established. |
| Transport by sea (IMDG) 14.5. Environmental hazards | : Not established. |
| Transport by road/rail/inland waterways (ADR/RID/ADN) | : None. |
| Transport by air (ICAO-TI / IATA-DGR) | : None. |
| Transport by sea (IMDG) 14.6. Special precautions for user | : None. |
| Packing Instruction(s) |  |
| Transport by road/rail/inland waterways (ADR/RID/ADN) | : P200 |
| Transport by air (ICAO-TI / IATA-DGR) |  |
| Passenger and Cargo Aircraft | : 200. |
| Cargo Aircraft only | : 200. |
| Transport by sea (IMDG) | : P200 |
| Special transport precautions | : Avoid transport on vehicles where the load space is not separated from the driver's compartment. Ensure vehicle driver is aware of the potential hazards of the load and knows what to do in the event of an accident or an emergency. Before transporting product containers: - Ensure there is adequate ventilation. - Ensure that containers are firmly secured. - Ensure valve is closed and not leaking. - Ensure valve outlet cap nut or plug (where provided) is correctly fitted. - Ensure valve protection device (where provided) is correctly fitted. |

14.7. Maritime transport in bulk according to IMO instruments

Not applicable.

# SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

## EU-Regulations

| Restrictions on use | : None. Contains no substance(s) listed on the REACH Candidate List |
| --- | --- |
| National legislation | : Ensure all national/local regulations are observed. |
| Seveso Directive : 2012/18/EU (Seveso III) | : Not covered. |

## National regulations

Ensure all national/local regulations are observed.

15.2. Chemical safety assessment

A CSA does not need to be carried out for this product.

# SECTION 16: Other information

Indication of changes 	: Safety data sheet in accordance with commission regulation (EU) No 2020/878.

| Section | Changed item | Change | Comments |
| --- | --- | --- | --- |
| 1.3 | Company | Modified | Version 6.0. New address in Sweden. (This change only applies to the Swedish (SE) version of this SDS) |

| Abbreviations and acronyms | : ATE - Acute Toxicity Estimate CLP - Classification Labelling Packaging Regulation; Regulation (EC) No 1272/2008 REACH - Registration, Evaluation, Authorisation and Restriction of Chemicals Regulation (EC) No 1907/2006 EINECS - European Inventory of Existing Commercial Chemical Substances CAS# - Chemical Abstract Service number PPE - Personal Protection Equipment LC50 - Lethal Concentration to 50 % of a test population RMM - Risk Management Measures PBT - Persistent, Bioaccumulative and Toxic vPvB - Very Persistent and Very Bioaccumulative STOT- SE : Specific Target Organ Toxicity - Single Exposure CSA - Chemical Safety Assessment EN - European Standard UN - United Nations ADR - Agreement concerning the International Carriage of Dangerous Goods by Road IATA - International Air Transport Association IMDG code - International Maritime Dangerous Goods RID - Regulations concerning the International Carriage of Dangerous Goods by Rail WGK - Water Hazard Class STOT - RE : Specific Target Organ Toxicity - Repeated Exposure UFI : Unique Formula Identifier |
| --- | --- |
| Training advice | : None. |
| Further information | : Classification in accordance with the procedures and calculation methods of Regulation (EC) 1272/2008 (CLP). |

| Full text of H- and EUH-statements |  |
| --- | --- |
| Ox. Gas 1 | Oxidising Gases, Category 1 |
| Press. Gas (Comp.) | Gases under pressure : Compressed gas |
| H270 | May cause or intensify fire; oxidiser. |
| H280 | Contains gas under pressure; may explode if heated. |

The classification complies with 	: ATP 12

DISCLAIMER OF LIABILITY 	: Before using this product in any new process or experiment, a thorough material

compatibility and safety study should be carried out.

Details given in this document are believed to be correct at the time of going to press. Whilst proper care has been taken in the preparation of this document, no liability for injury or damage resulting from its use can be accepted.

End of document

AIR

|  | Warning. |
| --- | --- |

Contains gas under pressure; may explode if heated

If medical advice is needed,  have product container or label at hand

Keep out of reach of children

Read carefully and follow all instructions

Protect from sunlight,  Store in a well-ventilated place

AirGas

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.

Nitrous Oxide

Safety Data Sheet 900027

according to Federal Register / Vol. 77, No. 58 / Monday, March 26, 2012 / Rules and Regulations

Date of issue: 01/29/2015

Supersedes: 09/10/2014

Version: 1.1

| SECTION 1: Identification |  |
| --- | --- |
| 1.1. Identification |  |
| Product form | : Substance |
| Substance name | : Nitrous Oxide |
| Chemical name | : Nitrous Oxide |
| CAS-No. | : 10024-97-2 |
| Product code | : SG-1001-00717 |
| Formula | : N2O |
| 1.2. Recommended use and restrictions on use |  |

Use of the substance/mixture	: Medical or Laboratory Purposes

| 1.3. | Supplier |
| --- | --- |

Air Liquide USA LLC and its affiliates

9811 Katy Freeway, Suite 100

Houston, TX 77024 - USA T 1-800-819-1704 www.us.airliquide.com

| 1.4. | Emergency telephone number |
| --- | --- |

Emergency number	: Chemtrec: 1-800-424-9300

# SECTION 2: Hazard(s) identification

| 2.1. | Classification of the substance or mixture |
| --- | --- |

GHS-US classification

| Oxidizing gases Category 1 | H270 | May cause or intensify fire; oxidizer |
| --- | --- | --- |
| Gases under pressure Liquefied gas | H280 | Contains gas under pressure; may explode if heated |
| Specific target organ toxicity (single exposure) Category 3 | H336 | May cause drowsiness or dizziness |

Full text of H statements : see section 16

| 2.2. | GHS Label elements, including precautionary statements |
| --- | --- |

GHS-US labeling

Hazard pictograms (GHS-US)	:

|  |  | GHS03 GHS04 GHS07 |
| --- | --- | --- |
| Signal word (GHS-US) | : | Danger |
| Hazard statements (GHS-US) | : | H270 - May cause or intensify fire; oxidizer H280 - Contains gas under pressure; may explode if heated H336 - May cause drowsiness or dizziness OSHA-H01 - May displace oxygen and cause rapid suffocation CGA-HG01 - May cause frostbite |
| Precautionary statements (GHS-US) | : | P202 - Do not handle until all safety precautions have been read and understood. P220 - Keep/Store away from clothing, Combustible materials P261 - Avoid breathing gas. P262 - Do not get in eyes, on skin, or on clothing. P271 - Use only outdoors or in a well-ventilated area. P315 - Get immediate medical advice/attention. P403 - Store in a well-ventilated place. P405 - Store locked up. P501 - Dispose of contents/container in accordance with local/regional/national/international regulations P244 - Keep reduction valves/valves and fittings free from oil and grease P304+P340 - If inhaled: Remove person to fresh air and keep comfortable for breathing P312 - Call a doctor if you feel unwell |

05/22/2018	EN (English US)	SDS ID: 900027	Page 1

P370+P376 - In case of fire: Stop leak if safe to do so.

P302+P336 - If on skin: Thaw frosted parts with lukewarm water. Do not rub affected area

CGA-PG02 - Protect from sunlight when ambient temperature exceeds 52°C/125 °F

CGA-PG05 - Use a back flow preventive device in the piping

CGA-PG06 - Close valve after each use and when empty

CGA-PG10 - Use only with equipment rated for cylinder pressure

CGA-PG14 - Approach suspected leak area with caution

CGA-PG20 - Use only with equipment of compatible materials of construction and rated for cylinder pressure

CGA-PG21 - Open valve slowly

CGA-PG22 - Use only with equipment cleaned for oxygen service

| 2.3. | Other hazards which do not result in classification |
| --- | --- |

Other hazards not contributing to the 	: None.

classification

| 2.4. | Unknown acute toxicity (GHS US) |
| --- | --- |

Not applicable

# SECTION 3: Composition/Information on ingredients

| 3.1. |  | Substances |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Substance type |  |  | : | Mono-constituent |
| Name |  |  | : | Nitrous Oxide |
| CAS-No. |  |  | : | 10024-97-2 |
| Name |  |  |  |  | Product identifier | % | GHS-US classification |
| Nitrous oxide |  |  |  |  | (CAS-No.) 10024-97-2 | > 99 | Ox. Gas 1, H270 Press. Gas (Liq.), H280 STOT SE 3, H336 |

Full text of hazard classes and H-statements : see section 16

| 3.2. Mixtures |  |
| --- | --- |
| Not applicable |  |
| SECTION 4: First-aid measures |  |
| 4.1. Description of first aid measures |  |
| First-aid measures after inhalation | : Remove victim to fresh air and keep at rest in a position comfortable for breathing. If you feel unwell, seek medical advice. |
| First-aid measures after skin contact | : Thaw frosted parts with lukewarm water. Do not rub affected area. Get immediate medical advice/attention. |
| First-aid measures after eye contact | : Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Consult an eye specialist. |
| First-aid measures after ingestion | : Ingestion is not considered a potential route of exposure. |
| 4.2. Most important symptoms and effects (acute and delayed) |  |

Symptoms/effects after inhalation	: May displace oxygen and cause rapid suffocation. May cause drowsiness or dizziness.

Symptoms/effects after skin contact	: Contact with the liquefied gas may cause frostbite.

Symptoms/effects after eye contact	: Contact with the product may cause cold burns or frostbite.

Symptoms/effects after ingestion	: Ingestion is not considered a potential route of exposure.

Symptoms/effects upon intravenous 	: Not known. administration

Chronic symptoms	: None known.

Most important symptoms and effects, both 	: Refer to section 11. acute and delayed

| 4.3. | Immediate medical attention and special treatment, if necessary |
| --- | --- |

If breathing is difficult, give oxygen. Obtain medical attention if breathing difficulty persists.

# SECTION 5: Fire-fighting measures

| 5.1. | Suitable (and unsuitable) extinguishing media |
| --- | --- |

Suitable extinguishing media	: Use extinguishing media appropriate for surrounding fire.

Unsuitable extinguishing media	: Do not use water jet to extinguish.

| 5.2. | Specific hazards arising from the chemical |
| --- | --- |

Fire hazard	: The product is not flammable.

| Explosion hazard | : |  | Product is not explosive. Heat may build pressure, rupturing closed containers, spreading fire and increasing risk of burns and injuries. |  |
| --- | --- | --- | --- | --- |
| Reactivity | : |  | None known. |  |
| Hazardous combustion products | : |  | None known |  |
| 5.3. Special protective equipment and precautions for fire-fighters |  |  |  |  |  |
| Firefighting instructions |  | : In case of fire: Evacuate area. Fight fire remotely due to the risk of explosion. Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. |  |  |  |
| Protection during firefighting |  | : Standard protective clothing and equipment (e.g, Self Contained Breathing Apparatus) for fire fighters. Do not enter fire area without proper protective equipment, including respiratory protection. |  |  |  |
| SECTION 6: Accidental release measures |  |  |  |  |  |
| 6.1. Personal precautions, protective equipment and emergency procedures |  |  |  |  |  |
| General measures |  | : |  | Ensure adequate ventilation. |  |
| 6.1.1. For non-emergency personnel Protective equipment |  | : |  | Wear protective equipment consistent with the site emergency plan. |  |
| Emergency procedures |  | : |  | Evacuate personnel to a safe area. Close doors and windows of adjacent premises. Keep containers closed. Mark the danger area. Seal off low-lying areas. Keep upwind. |  |
| 6.1.2. For emergency responders Protective equipment |  | : |  | Standard protective clothing and equipment (e.g, Self Contained Breathing Apparatus) for fire fighters. Equip cleanup crew with proper protection. |  |
| Emergency procedures |  | : |  | Evacuate and limit access. Ventilate area. |  |
| 6.2. Environmental precautions |  |  |  |  |  |
| Try to stop release if without risk. |  |  |  |  |  |
| 6.3. Methods and material for containment |  |  |  | and cleaning up |  |
| For containment |  | : |  | Try to stop release if without risk. |  |
| Methods for cleaning up |  | : |  | Dispose of contents/container in accordance with local/regional/national/international regulations. |  |
| Methods and material for containment and cleaning up |  | : |  | None. |  |
| 6.4. Reference to other sections |  |  |  |  |  |
| See also Sections 8 and 13. |  |  |  |  |  |
| SECTION 7: Handling and storage |  |  |  |  |  |
| 7.1. Precautions for safe handling |  |  |  |  |  |
| Additional hazards when processed |  | : |  | Pressurized container: Do not pierce or burn, even after use. Use only with equipment rated for cylinder pressure. |  |
| Precautions for safe handling |  | : |  | Do not handle until all safety precautions have been read and understood. Use only outdoors or in a well-ventilated area. |  |
| Hygiene measures |  | : |  | Do not eat, drink or smoke when using this product. |  |
| 7.2. Conditions for safe storage, including any incompatibilities |  |  |  |  |  |

Technical measures	: Comply with applicable regulations.

Storage conditions	: Store locked up. Do not expose to temperatures exceeding 52 °C/ 125 °F. Keep container

closed when not in use. Protect cylinders from physical damage; do not drag, roll, slide or drop.

Store in well ventilated area.

Incompatible products	: None known.

Incompatible materials	: Flammable materials. Reducing agents. Combustible materials.

Conditions for safe storage, including any 	: Observe all regulations and local requirements regarding storage of containers. Containers incompatibilities	should not be stored in conditions likely to encourage corrosion. Container valve guards or

caps should be in place. Containers should be stored in the vertical position and properly secured to prevent them from falling over. Stored containers should be periodically checked for general condition and leakage. Keep container below 50°C in a well ventilated place. Store containers in location free from fire risk and away from sources of heat and ignition. Keep away from combustible materials.

# SECTION 8: Exposure controls/personal protection

| 8.1. |  | Control parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nitrous oxide (10024-97-2) |  |  |  |  |  |  |
| ACGIH |  | ACGIH TWA (ppm) |  |  | 50 ppm |  |
| NIOSH |  | NIOSH REL (TWA) (mg/m³) |  |  | 46 mg/m³ (over the time exposed to waste anesthetic gas) |  |
| NIOSH |  | NIOSH REL (TWA) (ppm) |  |  | 25 ppm (over the time exposed to waste anesthetic gas) |  |
| 8.2. |  | Appropriate engineering controls |  |  |  |  |  |  |
| Appropriate engineering controls |  |  |  | : | Ensure exposure is below occupational exposure limits (where available). Provide adequate general and local exhaust ventilation. Systems under pressure should be regularly checked for leakages. Oxygen detectors should be used when asphyxiating gases may be released. Consider the use of a work permit system e.g. for maintenance activities. |  |
| Environmental exposure controls |  |  |  | : | Refer to local regulations for restriction of emissions to the atmosphere. See section 13 for specific methods for waste gas treatment. |  |
| 8.3. |  | Individual protection measures/Personal protective equipment |  |  |  |  |  |  |

Hand protection:

Wear working gloves when handling gas containers. 29 CFR 1910.138: Hand protection

Eye protection:

Wear safety glasses with side shields. 29 CFR 1910.133: Eye and Face Protection

Skin and body protection:

Wear suitable protective clothing, e.g. lab coats, coveralls or flame resistant clothing.

Respiratory protection:

None necessary during normal and routine operations.

Thermal hazard protection:

None necessary.

Other information:

Wear safety shoes while handling containers. 29 CFR 1910.136: Foot Protection.

# SECTION 9: Physical and chemical properties

| 9.1. | Information on basic physical and chemical properties |
| --- | --- |

Physical state	: Gas

Appearance	: Clear, colorless gas.

Color	: Colorless

Odor	: Slightly sweet

Odor threshold	: No data available

pH	: No data available

Melting point	: -90.81 °C

Freezing point	: -90.81 °C

Boiling point	: -87.45 °C

Critical temperature	: 37.45 °C

Critical pressure	: 7255 kPa

Flash point	: No data available

Relative evaporation rate (butyl acetate=1)	: No data available

Flammability (solid, gas)	: See Section 2.1 and 2.2

Non flammable. May cause or intensify fire; oxidizer.

| Vapor pressure | : | 30350.983591357 mbar |
| --- | --- | --- |
| Relative vapor density at 20 °C | : | 1.53 |
| Relative density | : | 1.2 |
| Specific gravity / density | : | 1.977 g/l |
| Molecular mass | : | 44.013 g/mol |
| Relative gas density | : | 1.5 |
| Solubility | : | Water: 0.1 % |
| Log Pow | : | Not applicable for gas-mixtures. Not applicable for gas-mixtures. |
| Auto-ignition temperature | : | No data available |
| Decomposition temperature | : | No data available |
| Viscosity, kinematic | : | Not applicable. |
| Viscosity, dynamic | : | Not applicable. |
| Explosion limits | : | Not applicable - not flammable |
| Explosive properties | : | Not flammable. |
| Oxidizing properties | : | Not combustible but enhances combustion of other substances. May intensify fire. Oxidizer. |
| Ci | : | 0.6 |
| 9.2. Other information |  |  |
| Gas group | : | Compressed gas |
| Additional information | : | Gas/vapor heavier than air. May accumulate in confined spaces, particularly at or below ground level |
| SECTION 10: Stability and reactivity |  |  |
| 10.1. Reactivity |  |  |
| None known. |  |  |
| 10.2. Chemical stability |  |  |
| Stable under normal conditions. |  |  |
| 10.3. Possibility of hazardous reactions |  |  |
| May react violently with reducing agents. |  |  |
| 10.4. Conditions to avoid |  |  |
| Refer to Section 10 on Incompatible Materials. |  |  |
| 10.5. Incompatible materials |  |  |

Flammable materials. Reducing agents. Combustible materials.

| 10.6. | Hazardous decomposition products |
| --- | --- |

Nitrous oxide explosively decomposes at elevated temperatures (above 1200 deg. F, 650 deg. C) into nitrogen and oxygen.

# SECTION 11: Toxicological information

| 11.1. |  | Information on toxicological effects |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Likely routes of exposure |  |  | : |  | Inhalation; Skin and eye contact |  |  |
| Acute toxicity |  |  | : |  | Not classified |  |  |
| Nitrous Oxide (10024-97-2) |  |  |  |  |  |  |  |  |
| LC50 inhalation rat (ppm) |  |  | 250000 ppm/4h |  |  |  |  |  |
| ATE US (gases) |  |  | 250000.000 ppmV/4h |  |  |  |  |  |
| Nitrous oxide (10024-97-2) |  |  |  |  |  |  |  |  |
| LC50 inhalation rat (ppm) |  |  | 250000 ppm/4h |  |  |  |  |  |
| Skin corrosion/irritation |  |  | : |  | Not classified |  |
| Serious eye damage/irritation |  |  | : |  | Not classified |  |
| Respiratory or skin sensitization |  |  | : |  | Not classified |  |
| Germ cell mutagenicity |  |  | : |  | Not classified |  |
| Carcinogenicity |  |  | : |  | Not classified |  |
| Reproductive toxicity |  |  | : |  | Not classified |  |
| Specific target organ toxicity – single exposure |  |  | : |  | May cause drowsiness or dizziness. |  |  |  |
| Specific target organ toxicity – repeated exposure |  |  | : |  | Not classified |  |  |  |
| Aspiration hazard |  |  | : |  | Not classified |  |  |  |
| Symptoms/effects after inhalation |  |  | : |  | May displace oxygen and cause rapid suffocation. May cause drowsiness or dizziness. |  |  |  |
| Symptoms/effects after skin contact |  |  | : |  | Contact with the liquefied gas may cause frostbite. |  |  |  |
| Symptoms/effects after eye contact |  |  | : |  | Contact with the product may cause cold burns or frostbite. |  |  |  |
| Symptoms/effects after ingestion |  |  | : |  | Ingestion is not considered a potential route of exposure. |  |  |  |
| Symptoms/effects upon intravenous administration |  |  | : |  | Not known. |  |  |  |
| Chronic symptoms |  |  | : |  | None known. |  |  |  |
| SECTION 12: Ecological information |  |  |  |  |  |  |  |  |  |  |
| 12.1. Toxicity |  |  |  |  |  |  |  |  |  |  |
| Ecology - general : |  |  |  |  |  | Classification criteria are not met. |  |  |  |  |
| 12.2. Persistence and degradability |  |  |  |  |  |  |  |  |  |  |
| Nitrous Oxide (10024-97-2) |  |  |  |  |  |  |  |  |
| Persistence and degradability |  |  | No data available. |  |  |  |  |  |
| Nitrous oxide (10024-97-2) |  |  |  |  |  |  |  |  |
| Persistence and degradability |  |  | Not applicable for inorganic gases. |  |  |  |  |  |
| 12.3. |  | Bioaccumulative potential |  |  |  |  |  |  |  |  |
| Nitrous Oxide (10024-97-2) |  |  |  |  |  |  |  |  |
| Log Pow |  |  | Not applicable for gas-mixtures. |  |  |  |  |  |
| Log Kow |  |  | Not applicable for gas-mixtures. |  |  |  |  |  |
| Bioaccumulative potential |  |  | No data available. |  |  |  |  |  |
| Nitrous oxide (10024-97-2) |  |  |  |  |  |  |  |  |
| Log Pow |  |  | Not applicable for inorganic gases. |  |  |  |  |  |
| Bioaccumulative potential |  |  | No data available. |  |  |  |  |  |
| 12.4. |  | Mobility in soil |  |  |  |  |  |  |  |  |
| Nitrous Oxide (10024-97-2) |  |  |  |  |  |  |  |  |
| Mobility in soil |  |  | No data available |  |  |  |  |  |
| Nitrous oxide (10024-97-2) |  |  |  |  |  |  |  |  |
| Ecology - soil |  |  | Because of its high volatility, the product is unlikely to cause ground or water pollution. |  |  |  |  |  |
| 12.5. |  | Other adverse effects |  |  |  |  |  |  |  |  |

Effect on ozone layer	: None

# SECTION 13: Disposal considerations

| 13.1. Disposal methods |  |  |
| --- | --- | --- |
| Waste treatment methods | : | Contact supplier if guidance is required. Do not discharge into any place where its accumulation could be dangerous. Ensure that the emission levels from local regulations or operating permits are not exceeded. |
| Product/Packaging disposal recommendations | : | Refer to the CGA Pamphlet P-63 "Disposal of Gases" available at www.cganet.com for more guidance on suitable disposal methods. |
| Additional information | : | None. |
| SECTION 14: Transport information |  |  |

Department of Transportation (DOT)

| In accordance with DOT |  |  |
| --- | --- | --- |
| Transport document description | : | UN1070 Nitrous oxide, 2.2 |
| UN-No.(DOT) | : | UN1070 |
| Proper Shipping Name (DOT) | : | Nitrous oxide |  |
| Class (DOT) | : | 2.2 - Class 2.2 - Non-flammable compressed gas 49 CFR 173.115 |  |
| Hazard labels (DOT) | : | 2.2 - Non-flammable gas 5.1 - Oxidiser |  |

| DOT Packaging Non Bulk (49 CFR 173.xxx) : |  | 304 |
| --- | --- | --- |
| DOT Packaging Bulk (49 CFR 173.xxx) : |  | 314;315 |
| DOT Special Provisions (49 CFR 172.102) : |  | A14 - This material is not authorized to be transported as a limited quantity or consumer commodity in accordance with 173.306 of this subchapter when transported aboard an aircraft. |
| DOT Packaging Exceptions (49 CFR 173.xxx) : |  | 306 |
| DOT Quantity Limitations Passenger aircraft/rail : (49 CFR 173.27) |  | 75 kg |
| DOT Quantity Limitations Cargo aircraft only (49 : |  | 150 kg |
| CFR 175.75) |  |  |
| DOT Vessel Stowage Location | : | A - The material may be stowed ‘‘on deck’’ or ‘‘under deck’’ on a cargo vessel and on a passenger vessel. |
| DOT Vessel Stowage Other | : | 40 - Stow ‘‘clear of living quarters’’ |
| Other information | : | No supplementary information available. |
| Special transport precautions Transportation of Dangerous Goods Transport by sea | : | Avoid transport on vehicles where the load space is not separated from the driver's compartment. Ensure vehicle driver is aware of the potential hazards of the load and knows what to do in the event of an accident or an emergency. Before transporting product containers: - Ensure there is adequate ventilation. - Ensure that containers are firmly secured. - Ensure cylinder valve is closed and not leaking. - Ensure valve outlet cap nut or plug (where provided) is correctly fitted. - Ensure valve protection device (where provided) is correctly fitted. |
| Transport document description (IMDG) | : | UN 1070 Nitrous oxide, 2 |
| UN-No. (IMDG) | : | 1070 |
| Proper Shipping Name (IMDG) | : | Nitrous oxide |
| Class (IMDG) | : | 2 - Gases |
| Limited quantities (IMDG) Air transport | : | 120 ml |
| Transport document description (IATA) | : | UN 1070 Nitrous oxide, 5.1 |
| UN-No. (IATA) | : | 1070 |
| Proper Shipping Name (IATA) | : | Nitrous oxide |
| Class (IATA) | : | 2.2 - Gases : Non-flammable, non-toxic |

# SECTION 15: Regulatory information

15.1. US Federal regulations

Nitrous oxide (10024-97-2)

Listed on the United States TSCA (Toxic Substances Control Act) inventory

15.2. International regulations

CANADA

Nitrous oxide (10024-97-2)

Listed on the Canadian DSL (Domestic Substances List)

EU-Regulations

Nitrous oxide (10024-97-2)

Listed on the EEC inventory EINECS (European Inventory of Existing Commercial Chemical Substances)

National regulations

Nitrous oxide (10024-97-2)

Listed on the AICS (Australian Inventory of Chemical Substances) Listed on IECSC (Inventory of Existing Chemical Substances Produced or Imported in China) Listed on the Japanese ENCS (Existing & New Chemical Substances) inventory Listed on the Japanese ISHL (Industrial Safety and Health Law) Listed on the Korean ECL (Existing Chemicals List) Listed on NZIoC (New Zealand Inventory of Chemicals) Listed on PICCS (Philippines Inventory of Chemicals and Chemical Substances) Listed on the Canadian IDL (Ingredient Disclosure List) Listed on INSQ (Mexican National Inventory of Chemical Substances) Listed on the TCSI (Taiwan Chemical Substance Inventory)

15.3. US State regulations

| Nitrous oxide (10024-97-2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| U.S. - California - Proposition 65 - Carcinogens List | U.S. - California - Proposition 65 - Developmental Toxicity | U.S. - California - Proposition 65 - Reproductive Toxicity - Female | U.S. - California - Proposition 65 - Reproductive Toxicity - Male | No significant risk level (NSRL) |
| No | Yes | Yes | No |  |
| Nitrous oxide (10024-97-2) |  |  |  |  |
| U.S. - Massachusetts - Right To Know List U.S. - New Jersey - Right to Know Hazardous Substance List U.S. - Pennsylvania - RTK (Right to Know) List |  |  |  |  |

# SECTION 16: Other information

Other information	: This Safety Data Sheet is offered pursuant to OSHA’s Hazard Communication Standard, 29

CFR, 1910.1200. Other government regulations must be reviewed for applicability to this product.

Full text of H-phrases:

| H270 | May cause or intensify fire; oxidizer |
| --- | --- |
| H280 | Contains gas under pressure; may explode if heated |
| H336 | May cause drowsiness or dizziness |

------

SDS US (GHS HazCom 2012)

This Safety Data Sheet is offered pursuant to OSHA’s Hazard Communication Standard, 29 CFR, 1910.1200. Other government regulations must be reviewed for applicability to this product. To the best of Air Liquide USA LLC and its affiliates' knowledge, the information contained herein is reliable and accurate as of this date; however, accuracy, suitability or completeness are not guaranteed and no warranties of any type, either express or implied, are provided. The information contained herein relates only to this specific product. If this product is combined with other materials, all component properties must be considered. Data may be changed from time to time. Be sure to consult the latest edition.

NITROUS OXIDE

|  | Danger. |
| --- | --- |

Contains gas under pressure; may explode if heated

May cause drowsiness or dizziness

May cause or intensify fire; oxidizer

Do not handle until all safety precautions have been read and understood

Keep/Store away from clothing/.../combustible materials

Keep valves and fittings free from oil and grease

Avoid breathing dust/fume/gas/mist/vapours/spray

Do not get in eyes,  on skin,  or on clothing Use only outdoors or with adequate ventilation IF ON SKIN:

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing

Call a POISON CENTER or doctor/physician if you feel unwell

Get immediate medical advice/attention

Thaw frosted parts with lukewarm water. Do no rub affected area

In case of fire: Stop leak if safe to do so

Store in a well-ventilated place

Store locked up

Dispose of contents/container to ...

Air Liquide

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.

Safety Data Sheet

Nitric Oxide

Red Ball Oxygen Co., Inc. P.O. Box 7316

Shreveport, LA 71137-7316

Phone: 318-425-3211 Fax: 318-425-6302 http://www.redballoxygen.com

# Section 1: Product and Company Identification

Red Ball Oxygen Co., Inc. P.O. Box 7316

Shreveport, LA 71137-7316

Phone: 318-425-3211 Fax: 318-425-6302 http://www.redballoxygen.com

Product Code: Nitric Oxide

# Section 2: Hazards Identification

Danger

Hazard Classification:

Corrosive To Metal (Category 1)

Gases Under Pressure

Oxidizing Gas (Category 1)

Hazard Statements:

Contains gas under pressure; may explode if heated

May be corrosive to metals

May cause or intensify fire; oxidizer

## Precautionary Statements

Prevention:

Keep reduction valves/valves and fittings free from oil and grease.

Keep only in original container.

Keep and store away from clothing and combustible materials.

Response:

Absorb spillage to prevent material damage.

In case of fire: Stop leak if safe to do so.

Storage:

Protect from sunlight.

Store in well-ventilated place.

Store in corrosive resistant container with a resistant inner liner.

# Section 3: Composition/Information on Ingredients

CAS #

10102-43-9

| Chemical Substance | Chemical Family | Trade Names |
| --- | --- | --- |
| NITRIC OXIDE | inorganic, gas | NITROGEN OXIDE (NO); NITRIC OXIDE (NO); NITRIC OXIDE TRIMER; NITROGEN MONOXIDE; NITROGEN MONOOXIDE; NITROGEN OXIDE (N4O4); NITROSYL RADICAL; RCRA P076; STCC 4920330; UN 1660; NO |

# Section 4: First Aid Measures

| Skin Contact | Eye Contact | Ingestion | Inhalation | Note to Physicians |
| --- | --- | --- | --- | --- |
| Wash skin with soap and water for at least 15 minutes while removing contaminated clothing and shoes. Get medical attention, if needed. Thoroughly clean and dry contaminated clothing and shoes before reuse. | Flush eyes with plenty of water for at least 15 minutes. Then get immediate medical attention. | If a large amount is swallowed, get medical attention. | If adverse effects occur, remove to uncontaminated area. Give artificial respiration if not breathing. Get immediate medical attention. | None |

# Section 5: Fire Fighting Measures

| Suitable Extinguishing Media | Products of Combustion | Protection of Firefighters |
| --- | --- | --- |
| Water Do not use dry chemicals, carbon dioxide or halogenated extinguishing agents. Large fires: Flood with fine water spray. | Nitrogen oxides | - Any self-contained breathing apparatus with a full facepiece. - Any self-contained breathing apparatus with a full facepiece. |

# Section 6: Accidental Release Measures

| Personal Precautions | Environmental Precautions | Methods for Containment Stop leak if possible without personal risk. |
| --- | --- | --- |
| Keep unnecessary people away, isolate hazard area and deny entry. Ventilate closed spaces before entering. Avoid contact with combustible materials. | Avoid contamination of water, soil, drains, and sewers. |  |

| Methods for Cleanup | Other Information |
| --- | --- |
| Contact emergency personnel. | Notify Local Emergency Planning Committee and State Emergency Response Commission for release greater than or equal to RQ (U.S. SARA Section 304). If release occurs in the U.S. and is reportable under CERCLA Section 103, notify the National Response Ceter at (800)424-8802 (USA) or (202)426-2675 (USA). |

# Section 7: Handling and Storage

| Handling | Storage |
| --- | --- |
| Handling | Storage |
| Store and handle in accordance with all current regulations and standards. NFPA 430 Code for the Storage of Liquid and Solid Oxidizing Materials. Notify State Emergency Response Commission for storage or use at amounts greater than or equal to the TPQ (U.S. EPA SARA Section 302). SARA Section 303 requires facilities storing a material with a TPQ to participate in local emergency response planning (U.S. EPA 40 CFR 355.30). | Keep separated from incompatible substances. |

# Section 8: Exposure Controls/Personal Protection

Exposure Guidelines

NITRIC OXIDE: 25 ppm (30 mg/m3) OSHA TWA 25 ppm ACGIH TWA 25 ppm (30 mg/m3) NIOSH recommended TWA 10 hour(s)

## Engineering Controls

Handle only in fully enclosed systems.

| Eye Protection | Skin Protection | Respiratory Protection |
| --- | --- | --- |
| Wear splash resistant safety goggles. Provide an emergency eye wash fountain and quick drench shower in the immediate work area. | Wear appropriate chemical resistant clothing. | Any self-contained breathing apparatus with a full facepiece. |

## General Hygiene considerations

- Avoid breathing vapor or mist

- Avoid contact with eyes and skin

- Wash thoroughly after handling and before eating or drinking

# Section 9: Physical and Chemical Properties

| Physical State | Appearance | Color | Change in Appearance | Physical Form | Odor | Taste |
| --- | --- | --- | --- | --- | --- | --- |
| Gas | Clear | Colorless | N/A | Gas | Not available | N/A |

| Flash Point | Flammability | Partition Coefficient | Autoignition Temperature | Upper Explosive Limits | Lower Explosive Limits Nonflammable |
| --- | --- | --- | --- | --- | --- |
| Not applicable | Not available | Not available | Nonflammable | Nonflammable |  |

| Boiling Point | Freezing Point | Vapor Pressure | Vapor Density | Specific Gravity | Water Solubility | pH | Odor Threshold | Evaporation Rate | Viscosity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -242 F (-152 C) | -263 F (- 164 C) | 26000 mmHg @ 20 C | 1.036 (Air=1) | Not applicable | 7.3% @ 0 C | Not applicable | 0.3-1.0 ppm | Not applicable | 0.0188 cP @ 25 C |

| Molecular Weight | Molecular Formula | Density | Weight per Gallon | Volatility by Volume | Volatility | Solvent Solubility |
| --- | --- | --- | --- | --- | --- | --- |
| 30.01 | N-O | 1.3402 g/L | Not available | Not available | Not applicable | Soluble: Sulfuric acid, alcohol, ferrous sulfate solutions, carbon disulfide |

# Section 10: Stability and Reactivity

| Stability | Conditions to Avoid | Incompatible Materials |
| --- | --- | --- |
| May react on contact with air. May react on contact with water. Releases toxic, corrosive, flammable or explosive gases. May explode during distillation or evaporation. | May react on contact with air. May react on contact with water. Releases toxic, corrosive, flammable or explosive gases. May explode during distillation or evaporation. | Metals, bases, metal oxides, reducing agents, combustible materials, halo carbons, oxidizing materials, halogens, metal carbide, metal salts |

| Hazardous Decomposition Products | Possibility of Hazardous Reactions |
| --- | --- |
| Oxides of nitrogen | Will not polymerize. |

# Section 11: Toxicology Information

## Acute Effects

| Oral LD50 | Dermal LD50 | Inhalation |
| --- | --- | --- |
| Oral LD50 | Dermal LD50 | Inhalation |
| LC50 Inhalation Gas. Rat 1068 mg/m³ 4 hours | Not available | Irritation, nausea, vomiting, stomach pain, chest pain, difficulty breathing, headache, dizziness, bluish skin color, lung congestion |

| Eye Irritation | Skin Irritation | Sensitization |
| --- | --- | --- |
| Irritation (possibly severe) | Irritation (possibly severe) | Harmful if inhaled, respiratory tract irritation, skin irritation, blood damage |

## Chronic Effects

| Carcinogenicity | Mutagenicity | Reproductive Effects | Developmental Effects |
| --- | --- | --- | --- |
| Not available | Available. | Not available | No data |

# Section 12: Ecological Information

## Fate and Transport

| Eco toxicity | Persistence / Degradability | Bioaccumulation / Accumulation | Mobility in Environment |
| --- | --- | --- | --- |
| Fish toxicity: Not available Invertibrate toxicity: Not available Algal toxicity: Not available Phyto toxicity: Not available Other toxicity: Not available | Readily biodegrades | Not available | Not expected to leach through the soil or the sediment. |

# Section 13: Disposal Considerations

Subject to disposal regulations: U.S. EPA 40 CFR 262. Hazardous Waste Number(s): D001. D003. Dispose in accordance with all applicable regulations.

# Section 14: Transportation Information

## U.S. DOT 49 CFR 172.101

| Proper Shipping Name | ID Number | Hazard Class or Division | Packing Group | Labeling Requirements | Passenger Aircraft or Railcar Quantity Limitations | Cargo Aircraft Only Quantity Limitations | Additional Shipping Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nitric oxide, compressed | UN1660 | 2.3 | Not applicable | 2.3; 5.1;8 | Forbidden | Forbidden | Toxic-Inhalation Hazard Zone A |

## Canadian Transportation of Dangerous Goods

| Shipping Name | UN Number | Class | Packing Group / Risk Group Not applicable |
| --- | --- | --- | --- |
| Nitric oxide, compressed | UN1660 | 2.3; 5.1; 8 |  |

# Section 15: Regulatory Information

## U.S. Regulations

| CERCLA Sections | SARA 355.30 | SARA 355.40 |
| --- | --- | --- |
| 10 LBS RQ | 100 LBS TPQ | 10 LBS RQ |

## SARA 370.21

| Acute | Chronic | Fire | Reactive | Sudden Release |
| --- | --- | --- | --- | --- |
| Yes | No | No | No | Yes |

SARA 372.65

Not regulated.

OSHA Process Safety

250 LBS TQ

## State Regulations

CA Proposition 65 Not regulated.

## Canadian Regulations

WHMIS Classification ACD1

## National Inventory Status

| US Inventory (TSCA) | TSCA 12b Export Notification | Canada Inventory (DSL/NDSL) |
| --- | --- | --- |
| Listed on inventory. | Not listed. | Not determined. |

# Section 16: Other Information

NFPA Rating

HEALTH=4 FIRE=0 REACTIVITY=1

0 = minimal hazard, 1 = slight hazard, 2 = moderate hazard, 3 = severe hazard, 4 = extreme hazard

NITRIC OXIDE

|  | Danger. |
| --- | --- |

Contains gas under pressure; may explode if heated

May be corrosive to metals

May cause or intensify fire; oxidizer

Keep only in original packaging

Keep valves and fittings free from oil and grease

In case of fire: Stop leak if safe to do so

Absorb spillage to prevent material damage

Store in accordance with…

Store in a corrosive resistant/... container with a resistant inner liner Protect from sunlight

Red Ball Oxygen

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.

Nitrogen Dioxide

Safety Data Sheet

according to Federal Register / Vol. 77, No. 58 / Monday, March 26, 2012 / Rules and Regulations

Date of issue: 03/25/2016

Supersedes: 07/22/2015

Version: 2.1

| SECTION 1: Identification |  |  |
| --- | --- | --- |
| 1.1. Identification |  |  |
| Product form | : | Substance |
| Substance name | : | Nitrogen Dioxide |
| CAS No | : | 10102-44-0 |
| Product code | : | SG-1001-02734 |
| Formula | : | NO2 |
| Synonyms | : | Nitrogen oxide (NO2) |
| Other means of identification | : | Dinitrogen Tetroxide Nitrogen Tetroxide Nitrogen Peroxide |
| 1.2. Relevant identified uses of the substance or mixture and uses advised against |  |  |

Use of the substance/mixture	: Test gas/Calibration gas

| 1.3. | Details of the supplier of the safety data sheet |
| --- | --- |

Air Liquide USA LLC and its affiliates

9811 Katy Freeway, Suite 100

Houston, TX 77024 - USA T 1-800-819-1704 www.us.airliquide.com

| 1.4. | Emergency telephone number |
| --- | --- |

Emergency number	: CHEMTREC:  1-800-424-9300

# SECTION 2: Hazard(s) identification

| 2.1. Classification of the substance or mixture |  |
| --- | --- |
| GHS-US classification Oxidizing gases Category 1 | H270 |
| Gases under pressure Liquefied gas | H280 |
| Acute toxicity (inhalation:gas) Category 1 | H330 |
| Skin corrosion/irritation Category 1B | H314 |
| Serious eye damage/eye irritation Category 1 | H318 |
| Specific target organ toxicity (single exposure) Category 2 Full text of H statements : s | H371 ee section 16 |
| 2.2. Label elements |  |

GHS-US labeling

Hazard pictograms (GHS-US)	:

|  |  | GHS03 GHS04 GHS05 GHS06 | GHS08 |
| --- | --- | --- | --- |
| Signal word (GHS-US) | : | Danger |  |
| Hazard statements (GHS-US) | : | H270 - May cause or intensify fire; oxidizer H280 - Contains gas under pressure; may explode if heated H314 - Causes severe skin burns and eye damage H318 - Causes serious eye damage H330 - Fatal if inhaled H371 - May cause damage to organs (lungs) CGA-HG11 - Symptoms may be delayed |  |

04/04/2016	EN (English US)	Page 1

CGA-HG22 - Corrosive to the respiratory tract

| Precautionary statements (GHS-US) |  |  | : | P202 - Do not handle until all safety precautions have been read and understood P220 - Keep/Store away from combustible materials, clothing P244 - Keep reduction valves/valves and fittings free from oil and grease P260 - Do not breathe gas P271 - Use only outdoors or in a well-ventilated area P280 - Wear eye protection, face protection, protective gloves, protective clothing P284 - Wear respiratory protection P303+P361+P353 - If on skin (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower P304+P340 - If inhaled: Remove person to fresh air and keep comfortable for breathing P305+P351+P338 - If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing P307+P311 - If exposed: Call a poison center/doctor P370+P376 - In case of fire: Stop leak if safe to do so P403 - Store in a well-ventilated place P405 - Store locked up P501 - Dispose of contents/container in accordance with local/regional/national/international regulations CGA-PG02 - Protect from sunlight when ambient temperature exceeds 52°C (125°F) CGA-PG05 - Use a back flow preventive device in the piping CGA-PG06 - Close valve after each use and when empty CGA-PG10 - Use only with equipment rated for cylinder pressure CGA-PG14 - Approach suspected leak area with caution CGA-PG18 - When returning cylinder, install leak tight valve outlet cap or plug CGA-PG20 - Use only with equipment of compatible materials of construction and rated for cylinder pressure CGA-PG21 - Open valve slowly CGA-PG22 - Use only with equipment cleaned for oxygen service |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.3. Other hazards |  |  |  |  |  |  |  |  |
| No additional information available |  |  |  |  |  |  |  |  |
| 2.4. Unknown acute toxicity (GHS US) |  |  |  |  |  |  |  |  |
| Not applicable |  |  |  |  |  |  |  |  |
| SECTION 3: Composition/Information on ingredients |  |  |  |  |  |  |  |  |
| 3.1. |  | Substance |  |  |  |  |  |  |
| Name |  |  |  | Product identifier | % | GHS-US classification |
| Nitrogen Dioxide (Main constituent) |  |  |  | (CAS No) 10102-44-0 | > 99 | Ox. Gas 1, H270 Liquefied gas, H280 Acute Tox. 1 (Inhalation:gas), H330 Skin Corr. 1B, H314 Eye Dam. 1, H318 STOT SE 2, H371 |

Full text of H-phrases: see section 16

| 3.2. Mixture |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Not applicable |  |  |  |  |  |
| SECTION 4: First aid measures |  |  |  |  |  |
| 4.1. Description of first aid measures |  |  |  |  |  |
| First-aid measures after inhalation |  | : |  | Remove victim to fresh air and keep at rest in a position comfortable for breathing. If not breathing, give artificial respiration. Apply artificial respiration with bag and mask if breathing stopped. Get immediate medical advice/attention. |  |
| First-aid measures after skin contact |  | : |  | Remove contaminated clothing. Drench affected area with water for at least 15 minutes. |  |
| First-aid measures after eye contact |  | : |  | Immediately flush eyes thoroughly with water for at least 15 minutes. |  |
| First-aid measures after ingestion |  | : |  | Ingestion is not considered a potential route of exposure. |  |
| 4.2. Most important symptoms and effects, both acute and delayed |  |  |  |  |  |
| Symptoms/injuries after inhalation | : |  | Fatal if inhaled. Corrosive to the respiratory tract. May cause damage to organs (lungs). |  |
| Symptoms/injuries after skin contact | : |  | Causes severe skin burns and eye damage. |  |
| Symptoms/injuries after eye contact | : |  | Causes serious eye damage. |  |
| Symptoms/injuries after ingestion | : |  | Ingestion is not considered a potential route of exposure. |  |
| Symptoms/injuries upon intravenous administration | : |  | Not known. |  |

Chronic symptoms	: Adverse effects not expected from this product.

| 4.3. | Indication of any immediate medical attention and special treatment needed |
| --- | --- |

If you feel unwell, seek medical advice. If breathing is difficult, give oxygen.

# SECTION 5: Firefighting measures

| 5.1. | Extinguishing media |
| --- | --- |

Suitable extinguishing media	: Use extinguishing media appropriate for surrounding fire.

Unsuitable extinguishing media	: Do not use water jet to extinguish.

| 5.2. Special hazards arising from the substance or mixture |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fire hazard |  |  |  | : | The product is not flammable. |  |  |
| Explosion hazard |  |  |  | : | Product is not explosive. Heat may build pressure, rupturing closed containers, spreading fire and increasing risk of burns and injuries. |  |  |
| 5.3. Advice for firefighters |  |  |  |  |  |  |  |
| Firefighting instructions |  |  |  | : | In case of fire: Evacuate area. Fight fire remotely due to the risk of explosion. Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. |  |  |
| Protection during firefighting |  |  |  | : | Standard protective clothing and equipment (e.g., Self Contained Breathing Apparatus) for fire fighters. Do not enter fire area without proper protective equipment, including respiratory protection. |  |  |
| SECTION 6: Accidental release measures |  |  |  |  |  |  |  |
| 6.1. Personal precautions, protective equipment and emergency procedures |  |  |  |  |  |  |  |
| 6.1.1. For non-emergency personnel Protective equipment |  |  |  | : Wear protective equipment consistent with the site emergency plan. |  |  |  |
| Emergency procedures |  |  |  | : | Escape the danger area by the closest safe route. Close doors and windows of adjacent premises. Keep containers closed. Mark the danger area. Seal off low-lying areas. Keep upwind. |  |  |
| 6.1.2. For emergency responders Protective equipment |  |  |  | : | Standard protective clothing and equipment (e.g., Self Contained Breathing Apparatus) for fire fighters. Equip cleanup crew with proper protection. |  |  |
| Emergency procedures |  |  |  | : | Evacuate and limit access. Ventilate area. |  |  |
| 6.2. Environmental precautions |  |  |  |  |  |  |  |
| Try to stop release if safe to do so. |  |  |  |  |  |  |  |
| 6.3. Methods and material for containment |  |  |  |  | and cleaning up |  |  |
| For containment |  |  |  | : | Try to stop release if safe to do so. |  |  |
| Methods for cleaning up |  |  |  | : | Dispose of this material and its container in accordance with local regulations. |  |  |
| 6.4. Reference to other sections |  |  |  |  |  |  |  |
| See also Sections 8 and 13. |  |  |  |  |  |  |  |
| SECTION 7: Handling and storage |  |  |  |  |  |  |  |
| 7.1. Precautions for safe handling |  |  |  |  |  |  |  |
| Additional hazards when processed |  |  |  | : | Pressurized container: Do not pierce or burn, even after use. Use equipment rated for cylinder pressure. Close valve after each use and when empty. |  |  |
| Precautions for safe handling |  |  |  | : | Do not handle until all safety precautions have been read and understood. Use only outdoors or in a well-ventilated area. |  |  |
| Hygiene measures |  |  |  | : | Do not eat, drink or smoke when using this product. |  |  |
| 7.2. Conditions for safe storage, including |  |  |  |  | any incompatibilities |  |  |
| Technical measures |  |  |  | : | Comply with applicable regulations. |  |  |
| Storage conditions |  |  |  | : | Do not expose to temperatures exceeding 52°C (125°F). Keep container closed when not in use. Protect cylinder from physical damage. Store in well ventilated area. Store locked up. |  |  |
| Incompatible products |  |  |  | : None known. |  |  |  |
| SECTION 8: Exposure controls/personal protection |  |  |  |  |  |  |  |
| 8.1. |  | Control parameters |  |  |  |  |  |
| Nitrogen Dioxide (10102-44-0) |  |  |  |  |  |
| ACGIH |  | ACGIH TWA (ppm) |  |  | 0.2 ppm |
| Nitrogen Dioxide (10102-44-0) |  |  |  |  |  |
| OSHA |  | OSHA PEL (Ceiling) (mg/m³) |  |  | 9 mg/m³ |
| OSHA |  | OSHA PEL (Ceiling) (ppm) |  |  | 5 ppm |
| IDLH |  | US IDLH (ppm) |  |  | 20 ppm |
| NIOSH |  | NIOSH REL (STEL) (mg/m³) |  |  | 1.8 mg/m³ |
| NIOSH |  | NIOSH REL (STEL) (ppm) |  |  | 1 ppm |
| 8.2. Exposure controls |  |  |  |  |  |  |  |
| Appropriate engineering controls |  |  |  | : | Ensure exposure is below occupational exposure limits. Provide adequate general and local exhaust ventilation. Systems under pressure should be regularly checked for leakages. Consider work permit system e.g. for maintenance activities. Alarm detectors should be used when toxic gases may be released. |  |  |
| Hand protection |  |  |  | : | Wear working gloves when handling gas containers. Standard EN 388 - Protective gloves against mechanical risk. Wear chemically resistant protective gloves. Standard EN 374 - Protective gloves against chemicals. Polyvinylchloride (PVC) /. Consult glove manufacturer´s product information on material suitability and material thickness. The breakthrough time of the selected gloves must be greater than the intended use period. |  |  |
| Eye protection |  |  |  | : | Wear safety glasses with side shields. Wear goggles and a face shield when transfilling or breaking transfer connections. Standard EN 166 - Personal eye-protection. Provide readily accessible eye wash stations and safety showers. |  |  |
| Skin and body protection |  |  |  | : | Wear suitable protective clothing, e.g. - lab coats, coveralls or flame resistant clothing. |  |  |
| Respiratory protection |  |  |  | : | Gas filters do not protect against oxygen deficiency. Keep self contained breathing apparatus readily available for emergency use. Gas filters may be used if all surrounding conditions e.g. type and concentration of the contaminant(s) and duration of use are known. Use gas filters and full face mask, where exposure limits may be exceeded for a short-term period, e.g. connecting or disconnecting containers. Recommended: Filter NO (blue). Consult respiratory device supplier´s product information for the selection of the appropriate device. Standard EN 14387 - Gas filter(s), combined filter(s) and full face mask - EN 136. Standard EN 137 - Selfcontained open-circuit compressed air breathing apparatus with full face mask. Self contained breathing apparatus is recommended, where unknown exposure may be expected, e.g. during maintenance activities on installation systems. |  |  |
| Thermal hazard protection |  |  |  | : | None necessary during normal and routine operations. |  |  |
| Environmental exposure controls |  |  |  | : | Refer to local regulations for restriction of emissions to the atmosphere. See section 13 for specific methods for waste gas treatment. |  |  |
| Other information |  |  |  | : | Wear safety shoes while handling containers. Keep suitable chemically resistant protective clothing readily available for emergency use. 29 CFR 1910.136: Foot Protection. |  |  |
| SECTION 9: Physical and chemical properties |  |  |  |  |  |  |  |
| 9.1. |  | Information on basic physical and chemical properties |  |  |  |  |  |

Physical state	: Gas

Appearance	: Reddish brown. Gas.

Color	: Reddish brown

Odor	: irritating/pungent odor

Odor threshold	: No data available

pH	: No data available

Melting point	: -11.2 °C

Freezing point	: -11.2 °C

Boiling point	: 22.15 °C

Critical temperature	: 158.85 °C

Critical pressure	: 10100 kPa

Flash point	: Not applicable.

Relative evaporation rate (butyl acetate=1)	: No data available

Relative evaporation rate (ether=1)	: Not applicable.

Flammability (solid, gas)	: See Section 2.1 and 2.2

Explosion limits	: Non flammable.

Explosive properties	: Not applicable - not flammable.

| Oxidizing properties | : | Not combustible but enhances combustion of other substances. May cause or intensify fire; oxidizer. |
| --- | --- | --- |
| Vapor pressure | : | 100 kPa |
| Relative density | : | 1.4 |
| Relative vapor density at 20 °C | : | 1.58 |
| Molecular mass | : | 46.01 g/mol |
| Relative gas density | : | 2.8 |
| Solubility | : | Water: Completely soluble. |
| Log Pow | : | Not applicable for inorganic gases. |
| Auto-ignition temperature | : | Not applicable. |
| Decomposition temperature | : | No data available |
| Viscosity | : | No data available |
| Viscosity, kinematic | : | Not applicable. |
| Viscosity, dynamic | : | Not applicable. |
| 9.2. Other information |  |  |
| Gas group | : | Liquefied gas |
| Additional information | : | Gas/vapor heavier than air. May accumulate in confined spaces, particularly at or below ground level |
| SECTION 10: Stability and reactivity |  |  |
| 10.1. Reactivity |  |  |
| No additional information available |  |  |
| 10.2. Chemical stability |  |  |
| Stable under normal conditions. |  |  |
| 10.3. Possibility of hazardous reactions |  |  |
| Violently oxidizes organic material. |  |  |
| 10.4. Conditions to avoid |  |  |
| Moisture. |  |  |
| 10.5. Incompatible materials |  |  |

May react violently with reducing agents. May react violently with combustible materials. Reacts with water to form corrosive acids. May react violently with alkalis. With water causes rapid corrosion of some metals.

| 10.6. | Hazardous decomposition products |
| --- | --- |

Under normal conditions of storage and use hazardous decomposition products should not be produced.

# SECTION 11: Toxicological information

| 11.1. | Information on toxicological effects |
| --- | --- |

Acute toxicity	: Inhalation:gas: Fatal if inhaled.

| Nitrogen Dioxide (10102-44-0) |  |
| --- | --- |
| LC50 inhalation rat (ppm) | 57.5 ppm/4h |
| ATE US (gases) | 57.500 ppmV/4h |

Skin corrosion/irritation	: Causes severe skin burns and eye damage.

pH: No data available

Serious eye damage/irritation	: Causes serious eye damage.

pH: No data available

Respiratory or skin sensitization	: Not classified

Germ cell mutagenicity	: Not classified

Carcinogenicity	: Not classified

Reproductive toxicity	: Not classified

Specific target organ toxicity (single exposure)	: May cause damage to organs (lungs).

Specific target organ toxicity (repeated 	Not classified exposure)

| Nitrogen Dioxide (10102-44-0) |  |
| --- | --- |
| Log Pow | Not applicable for inorganic gases. |
| Bioaccumulative potential | No data available. |

| Nitrogen Dioxide (10102-44-0) |  |
| --- | --- |
| Persistence and degradability | Not applicable for inorganic gases. |

| Nitrogen Dioxide (10102-44-0) |  |
| --- | --- |
| Ecology - soil | Because of its high volatility, the product is unlikely to cause ground or water pollution. |

| 12.4. | Mobility in soil |
| --- | --- |

| 12.5. | Other adverse effects |
| --- | --- |

| 12.3. | Bioaccumulative potential |
| --- | --- |

| SECTION 12: Ecological information |  |
| --- | --- |
| 12.1. Toxicity |  |
| Ecology - general : | No data available. |
| 12.2. Persistence and degradability |  |

| Aspiration hazard | : | Not classified |
| --- | --- | --- |
| Symptoms/injuries after inhalation | : | Fatal if inhaled. Corrosive to the respiratory tract. May cause damage to organs (lungs). |
| Symptoms/injuries after skin contact | : | Causes severe skin burns and eye damage. |
| Symptoms/injuries after eye contact | : | Causes serious eye damage. |
| Symptoms/injuries after ingestion | : | Ingestion is not considered a potential route of exposure. |
| Symptoms/injuries upon intravenous administration | : | Not known. |
| Chronic symptoms | : | Adverse effects not expected from this product. |

| Other adverse effects | : | May cause pH changes in aqueous ecological systems. |
| --- | --- | --- |
| Effect on ozone layer | : | No known effects from this product |

# SECTION 13: Disposal considerations

| 13.1. Waste treatment methods |  |  |
| --- | --- | --- |
| Waste treatment methods | : | Contact supplier if guidance is required. Do not discharge into any place where its accumulation could be dangerous. Ensure that the emission levels from local regulations or operating permits are not exceeded. |
| Waste disposal recommendations | : | Refer to the CGA Pamphlet P-63 "Disposal of Gases" available at www.cganet.com for more guidance on suitable disposal methods. |
| SECTION 14: Transport information |  |  |

Department of Transportation (DOT)

| In accordance with DOT |  |  |
| --- | --- | --- |
| Transport document description | : | UN1067 Dinitrogen tetroxide (Nitrogen Dioxide), 2.3 |
| UN-No.(DOT) | : | UN1067 |
| Proper Shipping Name (DOT) | : | Dinitrogen tetroxide |
| Class (DOT) | : | 2.3 - Class 2.3 - Poisonous gas 49 CFR 173.115 |
| Hazard labels (DOT) | : | 2.3 - Poison gas 5.1 - Oxidiser 8 - Corrosive |

| DOT Packaging Non Bulk (49 CFR 173.xxx) | : | 336 |
| --- | --- | --- |
| DOT Packaging Bulk (49 CFR 173.xxx) |  | 314 |

| DOT Special Provisions (49 CFR 172.102) | : | 1 - This material is poisonous by inhalation (see 171.8 of this subchapter) in Hazard Zone A (see 173.116(a) or 173.133(a) of this subchapter), and must be described as an inhalation hazard under the provisions of this subchapter B7 - Safety relief devices are not authorized on multi-unit tank car tanks. Openings for safety relief devices on multi-unit tank car tanks shall be plugged or blank flanged B14 - Each bulk packaging, except a tank car or a multi-unit-tank car tank, must be insulated with an insulating material so that the overall thermal conductance at 15.5 C (60 F) is no more than 1.5333 kilojoules per hour per square meter per degree Celsius (0.075 Btu per hour per square foot per degree Fahrenheit) temperature differential. Insulating materials must not promote corrosion to steel when wet B45 - Each tank must have a reclosing combination pressure relief device equipped with stainless steel or platinum rupture discs approved by the AAR Tank Car Committee B46 - The detachable protective housing for the loading and unloading valves of multi-unit tank car tanks must withstand tank test pressure and must be approved by the Associate Administrator B61 - Written procedures covering details of tank car appurtenances, dome fittings, safety devices, and marking, loading, handling, inspection, and testing practices must be approved by the Associate Administrator before any single unit tank car tank is offered for transportation B66 - Each tank must be equipped with gas tight valve protection caps. Outage must be sufficient to prevent tanks from becoming liquid full at 55 C (130 F). Specification 110A500W tanks must be stainless steel B67 - All valves and fittings must be protected by a securely attached cover made of metal not subject to deterioration by the lading, and all valve openings, except safety valve, must be fitted with screw plugs or caps to prevent leakage in the event of valve failure B77 - Other packaging are authorized when approved by the Associate Administrator T50 - When portable tank instruction T50 is referenced in Column (7) of the 172.101 Table, the applicable liquefied compressed gases are authorized to be transported in portable tanks in accordance with the requirements of 173.313 of this subchapter TP21 - The wall thickness must not be less than 8 mm. Portable tanks must be hydraulically tested and internally inspected at intervals not exceeding 2.5 years |
| --- | --- | --- |
| DOT Packaging Exceptions (49 CFR 173.xxx) | : | None |
| DOT Quantity Limitations Passenger aircraft/rail (49 CFR 173.27) | : | Forbidden |
| DOT Quantity Limitations Cargo aircraft only (49 : |  | Forbidden |
| CFR 175.75) |  |  |
| DOT Vessel Stowage Location | : | D - The material must be stowed ‘‘on deck only’’ on a cargo vessel and on a passenger vessel carrying a number of passengers limited to not more than the larger of 25 passengers or one passenger per each 3 m of overall vessel length, but the material is prohibited on passenger vessels in which the limiting number of passengers is exceeded |
| DOT Vessel Stowage Other | : | 40 - Stow ‘‘clear of living quarters’’,89 - Segregation same as for oxidizers,90 - Stow ‘‘separated from’’ radioactive materials |
| Emergency Response Guide (ERG) Number | : | 124 |
| Other information | : | No supplementary information available. |
| Special transport precautions TDG No additional information available Transport by sea | : | Avoid transport on vehicles where the load space is not separated from the driver's compartment. Ensure vehicle driver is aware of the potential hazards of the load and knows what to do in the event of an accident or an emergency. Before transporting product containers: - Ensure there is adequate ventilation. - Ensure that containers are firmly secured. - Ensure cylinder valve is closed and not leaking. - Ensure valve outlet cap nut or plug (where provided) is correctly fitted. - Ensure valve protection device (where provided) is correctly fitted. |
| UN-No. (IMDG) | : | 1067 |
| Proper Shipping Name (IMDG) | : | Dinitrogen Tetroxide |
| Class (IMDG) | : | 2 - Gases |
| MFAG-No Air transport | : | 124 |
| UN-No. (IATA) | : | 1067 |
| Proper Shipping Name (IATA) | : | Dinitrogen Tetroxide |
| Class (IATA) |  | 2 |

# SECTION 15: Regulatory information

15.1. US Federal regulations

| Nitrogen Dioxide (10102-44-0) |  |
| --- | --- |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory Listed on the United States SARA Section 302 |  |
| SARA Section 302 Threshold Planning Quantity (TPQ) | 100 |

15.2. International regulations

CANADA

| Nitrogen Dioxide (10102-44-0) |  |
| --- | --- |
| Listed on the Canadian DSL (Domestic Substances List) |  |
| WHMIS Classification | Class A - Compressed Gas Class C - Oxidizing Material Class D Division 1 Subdivision A - Very toxic material causing immediate and serious toxic effects Class D Division 2 Subdivision B - Toxic material causing other toxic effects Class E - Corrosive Material |

EU-Regulations

Nitrogen Dioxide (10102-44-0)

Listed on the EEC inventory EINECS (European Inventory of Existing Commercial Chemical Substances)

National regulations

| Nitrogen Dioxide (10102-44-0) |  |
| --- | --- |
| Listed on the AICS (Australian Inventory of Chemical Substances) Listed on IECSC (Inventory of Existing Chemical Substances Produced or Imported in China) Listed on the Japanese ENCS (Existing & New Chemical Substances) inventory Listed on the Korean ECL (Existing Chemicals List) Listed on NZIoC (New Zealand Inventory of Chemicals) Listed on PICCS (Philippines Inventory of Chemicals and Chemical Substances) Listed on the Canadian IDL (Ingredient Disclosure List) |  |
| 15.3. US State regulations |  |  |  |
| Nitrogen Dioxide (10102-44-0) |  |  |  |
| State or local regulations |  | U.S. - Massachusetts - Right To Know List U.S. - New Jersey - Right to Know Hazardous Substance List U.S. - Pennsylvania - RTK (Right to Know) - Environmental Hazard List U.S. - Pennsylvania - RTK (Right to Know) List |  |

# SECTION 16: Other information

Full text of H-phrases:

| H270 | May cause or intensify fire; oxidizer |
| --- | --- |
| H280 | Contains gas under pressure; may explode if heated |
| H314 | Causes severe skin burns and eye damage |
| H318 | Causes serious eye damage |
| H330 | Fatal if inhaled |
| H371 | May cause damage to organs |

------

------

------

------

------

------

| NFPA health hazard; : 3 - Short exposure could cause serious temporary or residual injury even though prompt medical attention was given.. NFPA fire hazard; : 0 - Materials that will not burn.. NFPA reactivity; : 0 - Normally stable, even under fire exposure conditions, and are not reactive with water.. NFPA specific hazard; : OX - This denotes an oxidizer, a chemical which can greatly increase the rate of combustion/fire.. |  |
| --- | --- |

SDS US (GHS HazCom 2012)

This Safety Data Sheet is offered pursuant to OSHA’s Hazard Communication Standard, 29 CFR, 1910.1200. Other government regulations must be reviewed for applicability to this product. To the best of Air Liquide USA LLC and its affiliates' knowledge, the information contained herein is reliable and accurate as of this date; however, accuracy, suitability or completeness are not guaranteed and no warranties of any type, either express or implied, are provided. The information contained herein relates only to this specific product. If this product is combined with other materials, all component properties must be considered. Data may be changed from time to time. Be sure to consult the latest edition.

NITROGEN DIOXIDE

|  | Danger. |
| --- | --- |

Causes severe skin burns and eye damage

Fatal if inhaled

Harmful to aquatic life with long lasting effects

May cause or intensify fire; oxidizer

Keep/Store away from clothing/.../combustible materials

Keep valves and fittings free from oil and grease

Do not breathe dust/fume/gas/mist/vapours/spray

Wash ... thoroughly after handling

Use only outdoors or with adequate ventilation

Avoid release to the environment

Wear protective gloves/protective clothing/eye protection/face protection

In case of inadequate ventilation wear respiratory protection

IF SWALLOWED: Rinse mouth,  Do NOT induce vomiting

IF ON SKIN (or hair): Take off immediately all contaminated clothing,  Rinse skin with water/shower

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing IF IN EYES: Rinse cautiously with water for several minutes,  Remove contact lenses,  if present and easy to do. Continue rinsing

Immediately call a POISON CENTER/doctor/…

Wash contaminated clothing before reuse

In case of fire: Stop leak if safe to do so

Store in a well-ventilated place,  Keep container tightly closed

Store locked up

Dispose of contents/container to ...

Aldrich

The information contained herein is based on data compiled from the chemical components of the (M)SDS and may not accurately represent the safety hazards for the product.  Only the manufacturer of the product can make actual representations about the hazard profile of a chemical product. No warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use thereof.